RTL9075AAD-V2-V3-GR

RTL9072AAD-V2-V3-GR

RTL9068AAD-V2-V3-GR

RTL9075AAD-VA-GR

RTL9072AAD-VA-GR

RTL9068AAD-VA-GR

SINGLE-CHIP AUTOMOTIVE ETHERNET SWITCH CONTROLLER WITH 100BASE-T1/1000BASE-T1 TRANSCEIVER

Automotive Switch Tool JSON Configuration Datasheet

(CONFIDENTIAL: Development Partners Only)

Rev. 1.12

6 Aug 2021

|  | Realtek Semiconductor Corp. No. 2, Innovation Road II, Hsinchu Science Park, Hsinchu 300, Taiwan Tel.: +886-3-578-0211. Fax: +886-3-577-6047 www.realtek.com |
| --- | --- |

COPYRIGHT

©2017 Realtek Semiconductor Corp. All rights reserved. No part of this document may be reproduced, transmitted, transcribed, stored in a retrieval system, or translated into any language in any form or by any means without the written permission of Realtek Semiconductor Corp.

DISCLAIMER

Realtek provides this document 'as is', without warranty of any kind. Realtek may make improvements and/or changes in this document or in the product described in this document at any time. This document could include technical inaccuracies or typographical errors.

TRADEMARKS

Realtek is a trademark of Realtek Semiconductor Corporation. Other names mentioned in this document are trademarks/registered trademarks of their respective owners.

LICENSE

This product is covered by one or more of the following patents: US5,307,459, US5,434,872, US5,732,094, US6,570,884, US6,115,776, and US6,327,625.

USING THIS DOCUMENT

This document is intended for the software engineer’s reference and provides detailed programming information.

Though every effort has been made to ensure that this document is current and accurate, more information may have become available subsequent to the production of this guide.

REVISION HISTORY

| Revision | Release Date | Summary |
| --- | --- | --- |
| 1.0 | 2020/03/31 | Formal release |
| 1.1 | 2020/09/30 | - Add "2.2 TabStacking" and "2.3 GlobalPtp9075" - Modify "2.8 GlobalAcl906x", "2.10 GlobalVlan" and "3.3 PortQos" |
| 1.2 | 2020/10/16 | - Modify "2.10 GlobalVlan" |
| 1.3 | 2020/11/06 | - Modify "2.4 GlobalMirror906x" - Modify "2.1 TabGeneral" - Add "2.6 GlobalSleep" |
| 1.4 | 2020/12/02 | - Add the special note in "1.2 Comparisons between V3 and VA" - Add "2.8.4 EntryAclAggreList" of "2.8 GlobalAcl906x" |
| 1.5 | 2020/12/11 | - Modify "2.7 GlobalReg" - Modify "2.4 GlobalMirror906x" - Add "3.5 PortStormControl" |
| 1.6 | 2020/12/31 | - Modify "2.6 GlobalSleep" - Add objects of "3.1 PortGeneral" - Add objects of "2.4 GlobalMirror906x" - Add "3.6 PortIsolation" |
| 1.7 | 2021/01/15 | - Add objects of "2.12 GlobalRouting" |
| 1.8 | 2021/02/04 | - Add objects of "2.3 GlobalPtp9075" - Add objects of "2.4 GlobalMirror906x" - Add objects of "2.12 GlobalRouting" |
| 1.9 | 2021/02/26 | - Add a field to describe the attribute of the object - Remove the unnecessary objects of "2.4 GlobalMirror906x" - Revise "2.3 GlobalPtp9075", "2.4 GlobalMirror906x", "2.8.5 EntryAclList", "3.1 PortGeneral" and "3.4 PortTraffic" - Add "2.14 GlobalFrer", "2.15 GlobalPsfp" and "2.16 GlobalPhyMaster" |
| 1.10 | 2021/03/19 | - Change the attributes of objects in "3.1 PortGeneral" - Add the objects of "3.2 PortVlan" |
| 1.11 | 2021/05/27 | - Add the objects of "2.3 GlobalPtp9075" - Add the objects of "3.1 PortGeneral" |
| 1.12 | 2021/08/06 | - Add objects of "2.12 GlobalRouting" |

Table of Contents

1	General Description	6

1.1	Structure	6

1.2	Comparisons between V3 and VA	7

2	Global Configuration	22

2.1	TabGeneral	22

2.2	TabStacking	23

2.3	GlobalPtp9075	24

2.4	GlobalMirror906x	27

2.5	GlobalMisc906x	30

2.6	GlobalSleep	31

2.7	GlobalReg	34

2.8	GlobalAcl906x	36

2.8.1	EntryAclUdfList	36

2.8.2	EntryAclRangeList	37

2.8.3	EntryAclGroupList	37

2.8.4	EntryAclAggreList	38

2.8.5	EntryAclList	38

2.8.6	Template Generator	40

2.9	GlobalLut	42

2.10	GlobalVlan	43

2.11	GlobalQos	44

2.11.1	Priority Decision	45

2.11.2	Priority Remarking	46

2.11.3	Priority Decision/Remarking List JSON Object	46

2.12	GlobalRouting	48

2.12.1	RoutingInterfaceList (Interface Config)	51

2.12.2	RoutingStaticList (Static Route)	51

2.12.3	RoutingMultiList (Multicast)	51

2.12.4	RoutingUniList (Global Routing Table)	52

2.12.5	RoutingHostList (Static Host)	52

2.12.6	RoutingNetworkList (Network List)	53

2.13	GlobalFlowControl	53

2.14	GlobalFrer	54

2.15	GlobalPsfp	55

2.16	GlobalPhyMaster	60

2.17 GlobalIgmp	60

3	Port	62

3.1	PortGeneral	62

3.2	PortVlan	64

3.3	PortQos	65

3.4	PortTraffic	68

3.5	PortStormControl	69

3.6	PortIsolation	70

General Description

Automotive Switch Tool JSON configuration file is used to save all configured settings on Automotive Switch Tool. Users can easily restore all the settings by importing this JSON file every time when the tool is opened. This JSON file can also be modified manually without opening the tool so that external application can generate/modify the content by following the format if any requirement of switch setting is changed.

This document introduces the JSON format that Automotive Switch Tool can recognize and process, supported switch and tool version:

| Automotive Ethernet Switch | RTL9075AV2/RTL9072AV2/RTL9068AV2 RTL9075AV3/RTL9072AV3/RTL9068AV3 |
| --- | --- |
| Automotive Switch Tool | v2.1.5.17.0 - v2.1.5.26.0 |

| Automotive Ethernet Switch | RTL9075VA/RTL9072VA/RTL9068VA |
| --- | --- |
| Automotive Switch Tool | v2.1.6.0.0 - v2.1.6.20.0 |

Structure

Thru editing JSON file, the functions listed below are configurable:

| Settings | Properties in JSON |
| --- | --- |
| Chip version, interface | TabGeneral |
| Stacking | TabStacking |
| Global Settings: Mirroring | GlobalMirror906x |
| Global Settings: PTP | GlobalPtp9075 |
| Global Settings: Sleep and Wakeup | GlobalSleep |
| Global Settings: Test Mode | GlobalMisc906x |
| Global Settings: Register value modification | GlobalReg |
| Global Settings: Routing | GlobalRouting |
| Global Settings: ACL | GlobalAcl906x |
| Global Settings: L2 table | GlobalLut |
| Global Settings: VLAN | GlobalVlan |
| Global Settings: QoS | GlobalQos |
| Global Settings: Flow Control | GlobalFlowControl |
| Global Settings: 802.1CB Frame Replication and Elimination for Reliability (Stream Identification) | GlobalFrer |
| Global Settings: 802.1Qci Per-Stream Filtering and Policing | GlobalPsfp |
| Port Settings: Port Properties (Port interface, PHY role, RGMII Tx/Rx delay, xMII LDO Voltage, etc) | PortGeneral |
| Port Settings: VLAN | PortVlan |
| Port Settings: QoS | PortQos |
| Port Settings: 802.1Qbv Enhancements for Scheduled Traffic | PortTraffic |
| Port Settings: Storm Control | PortStormControl |
| Port Settings: Port Isolation | PortIsolation |

The diagram shows global structure of JSON configuration file. Please follow this structure to generate/modify JSON configuration. The format details of each function are showed in following chapters.

Comparisons between V3 and VA

Note: The two versions of JSON files can't be mixed. If mixed, there will be something wrong with this case.

|  |  |  |  |  |  | VA v2.1.6.15.0 | V3 v2.1.5.25.0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ViewModels | Global | TabGeneral | SavedChip |  |  | o | o |
|  | Global_S1 |  | SavedIF |  |  | o | o |
|  | Global_S2 |  | ConfigOutputPath |  |  | o | o |
|  | Global_S3 |  | JSON_Version |  |  | o | o |
|  |  |  | EtherDis |  |  | o | x |
|  |  | TabStacking | GroupID |  |  | o | o |
|  |  |  | EntryStackingList | Model |  | o | o |
|  |  |  |  | MAC Address |  | o | o |
|  |  |  |  | isMaster |  | o | o |
|  |  |  |  | Unit ID |  | o | o |
|  |  |  |  | S0 Link To |  | o | o |
|  |  |  |  | S1 Link To |  | o | o |
|  |  | GlobalMirror906x | MirroredRxPorts |  |  | x | o |
|  |  |  | MirroredTxPorts |  |  | x | o |
|  |  |  | BlockedFwPortMask |  |  | x | o |
|  |  |  | SelectedPortBasedMirrorNo |  |  | x | o |
|  |  |  | SelectedPortBasedMirrorPort |  |  | x | o |
|  |  |  | SelectedMirrorNo |  |  | x | o |
|  |  |  | IsMirrorFilterOn |  |  | x | o |
|  |  |  | SelectedPortBasedMirrorPort_1 |  |  | o | o |
|  |  |  | MirroredRxPorts_1 |  |  | o | o |
|  |  |  | MirroredTxPorts_1 |  |  | o | o |
|  |  |  | IsMirrorFilterOn_1 |  |  | o | o |
|  |  |  | InputSampleRate_1 |  |  | o | o |
|  |  |  | SelectedRSPAN_Action_1 |  |  | o | o |
|  |  |  | InputTPID_1 |  |  | o | o |
|  |  |  | InputCFI_1 |  |  | o | o |
|  |  |  | InputTagVID_1 |  |  | o | o |
|  |  |  | IsRSPAN_Check_1 |  |  | o | o |
|  |  |  | InputCheckVID_1 |  |  | o | o |
|  |  |  | SelectedPortBasedMirrorPort_2 |  |  | o | o |
|  |  |  | MirroredRxPorts_2 |  |  | o | o |
|  |  |  | MirroredTxPorts_2 |  |  | o | o |
|  |  |  | IsMirrorFilterOn_2 |  |  | o | o |
|  |  |  | InputSampleRate_2 |  |  | o | o |
|  |  |  | SelectedRSPAN_Action_2 |  |  | o | o |
|  |  |  | InputTPID_2 |  |  | o | o |
|  |  |  | InputCFI_2 |  |  | o | o |
|  |  |  | InputTagVID_2 |  |  | o | o |
|  |  |  | IsRSPAN_Check_2 |  |  | o | o |
|  |  |  | InputCheckVID_2 |  |  | o | o |
|  |  |  | IsMirrorTopologyEnable_1 |  |  | o | x |
|  |  |  | EgressPortsEnable_1 |  |  | o | x |
|  |  |  | MirroringTopologySet_1 |  |  | o | x |
|  |  |  | IsMirrorTopologyEnable_2 |  |  | o | x |
|  |  |  | EgressPortsEnable_2 |  |  | o | x |
|  |  |  | MirroringTopologySet_2 |  |  | o | x |
|  |  |  | MirrorTopologyEnable_1 |  |  | o | x |
|  |  |  | MirroringTopologyTxt_1 |  |  | o | x |
|  |  |  | MirrorTopologyEnable_2 |  |  | o | x |
|  |  |  | MirroringTopologyTxt_2 |  |  | o | x |
|  |  | GlobalPtp9075 | Slave0JoinedPtpPorts |  |  | o | o |
|  |  |  | Slave1JoinedPtpPorts |  |  | o | o |
|  |  |  | Slave2JoinedPtpPorts |  |  | o | o |
|  |  |  | SelectedEnableSync |  |  | o | o |
|  |  |  | SelectedBasicMode |  |  | o | o |
|  |  |  | SelectedSlave0 |  |  | o | o |
|  |  |  | SelectedSlave1 |  |  | o | o |
|  |  |  | SelectedSlave2 |  |  | o | o |
|  |  |  | Slave0Domain |  |  | o | o |
|  |  |  | Slave1Domain |  |  | o | o |
|  |  |  | Slave2Domain |  |  | o | o |
|  |  |  | Slave0Pri |  |  | o | o |
|  |  |  | Slave1Pri |  |  | o | o |
|  |  |  | Slave2Pri |  |  | o | o |
|  |  |  | SyncInterval |  |  | o | o |
|  |  |  | pdelayReq |  |  | o | o |
|  |  |  | SelectedGPIOB1 |  |  | o | o |
|  |  |  | SelectedGPIOB2 |  |  | o | o |
|  |  |  | GPIOB1 |  |  | o | o |
|  |  |  | GPIOB2 |  |  | o | o |
|  |  |  | SendPdelayReq |  |  | o | x |
|  |  |  | AssignPriority |  |  | o | x |
|  |  |  | AsCapableMaskDef |  |  | o | x |
|  |  |  | DelayTh_Sec |  |  | o | x |
|  |  |  | DelayTh_NanoSec |  |  | o | x |
|  |  |  | PortNumRemappingEnable |  |  | o | x |
|  |  |  | PortNumRemapping |  |  | o | x |
|  |  |  | OneStepTx0 |  |  | o | x |
|  |  |  | OneStepTx1 |  |  | o | x |
|  |  |  | OneStepTx2 |  |  | o | x |
|  |  |  | SignalEnableDomain0 |  |  | o | x |
|  |  |  | SignalEnableDomain1 |  |  | o | x |
|  |  |  | SignalEnableDomain2 |  |  | o | x |
|  |  |  | VlanSupport0 |  |  | o | x |
|  |  |  | VlanSupport1 |  |  | o | x |
|  |  |  | VlanSupport2 |  |  | o | x |
|  |  |  | VlanSupportVID0 |  |  | o | x |
|  |  |  | VlanSupportVID1 |  |  | o | x |
|  |  |  | VlanSupportVID2 |  |  | o | x |
|  |  |  | PacketDestAddrEnable |  |  | o | x |
|  |  |  | PacketDestAddr |  |  | o | x |
|  |  |  | PacketEtherTypeEnable |  |  | o | x |
|  |  |  | PacketEtherType |  |  | o | x |
|  |  |  | NoSyncRecovery |  |  | o | x |
|  |  |  | OverwriteClockID |  |  | o | x |
|  |  | GlobalSleep | SelectedSleepWakeConfig |  |  | o | x |
|  |  |  | WholeChipSleep |  |  | o | x |
|  |  |  | SelectedOPLocalState |  |  | o | x |
|  |  |  | SelectedOPGlobalState |  |  | o | x |
|  |  |  | SelectedRemoteWake |  |  | o | x |
|  |  |  | SelectedRemotWakePorts |  |  | o | x |
|  |  |  | SleepWakePorts |  |  | o | x |
|  |  |  | OPLocalStatePorts |  |  | o | x |
|  |  |  | RemoteWakePorts |  |  | o | x |
|  |  |  | TC10SC_Ports |  |  | o | x |
|  |  |  | AcceptLP_Ports |  |  | o | x |
|  |  |  | ForwardSR_Ports |  |  | o | x |
|  |  |  | WholeChipS_Ports |  |  | o | x |
|  |  |  | ForwardLP_Ports |  |  | o | x |
|  |  |  | DisableRWD_Ports |  |  | o | x |
|  |  |  | RemoteWakeEnable |  |  | o | x |
|  |  |  | RemoteWakePorts_0 |  |  | o | x |
|  |  |  | RemoteWakePorts_1 |  |  | o | x |
|  |  |  | RemoteWakePorts_2 |  |  | o | x |
|  |  |  | RemoteWakePorts_3 |  |  | o | x |
|  |  |  | RemoteWakePorts_4 |  |  | o | x |
|  |  |  | RemoteWakePorts_5 |  |  | o | x |
|  |  |  | RemoteWakePorts_6 |  |  | o | x |
|  |  |  | RemoteWakePorts_7 |  |  | o | x |
|  |  |  | RemoteWakePorts_8 |  |  | o | x |
|  |  |  | RemoteWakePorts_9 |  |  | o | x |
|  |  |  | RemoteWakePorts_10 |  |  | o | x |
|  |  | GlobalMisc906x | PhyId |  |  | o | o |
|  |  |  | TestMode |  |  | o | o |
|  |  |  | Is1000BaseT1 |  |  | o | o |
|  |  |  | Is1000BaseT1Master |  |  | o | o |
|  |  | GlobalReg | AddRegType |  |  | o | o |
|  |  |  | EntryRegList | RegAddr |  | o | o |
|  |  |  |  | DefaultRegVal |  | o | o |
|  |  |  |  | ConfigRegVal |  | o | o |
|  |  |  |  | RegPage |  | o | x |
|  |  |  |  | SgType |  | o | x |
|  |  |  |  | IsPageEnabled |  | o | x |
|  |  |  |  | EnabledPorts |  | o | x |
|  |  |  |  | RegType |  | o | o |
|  |  | GlobalRouting | RoutingInterfaceList | Index |  | o | o |
|  |  |  |  | Ip |  | o | o |
|  |  |  |  | IpType |  | o | o |
|  |  |  |  | Mask |  | o | o |
|  |  |  |  | Port |  | o | o |
|  |  |  |  | Vid |  | o | o |
|  |  |  |  | Mac |  | o | o |
|  |  |  | RoutingHostList | Type |  | o | o |
|  |  |  |  | Source |  | o | o |
|  |  |  |  | Destination |  | o | o |
|  |  |  |  | IpType |  | o | o |
|  |  |  |  | SPort |  | o | o |
|  |  |  |  | DPort |  | o | o |
|  |  |  |  | InterfaceIdx |  | o | o |
|  |  |  |  | NextHop |  | o | o |
|  |  |  |  | TTL |  | o | o |
|  |  |  |  | L2Index |  | o | o |
|  |  |  |  | Firewall |  | o | o |
|  |  |  | RoutingStaticList | Index |  | o | o |
|  |  |  |  | Ip |  | o | o |
|  |  |  |  | IpType |  | o | o |
|  |  |  |  | Mask |  | o | o |
|  |  |  |  | NextHopIp |  | o | o |
|  |  |  | RoutingMultiList | Index |  | o | o |
|  |  |  |  | Sip |  | o | o |
|  |  |  |  | Dip |  | o | o |
|  |  |  |  | IpType |  | o | o |
|  |  |  |  | SPort |  | o | o |
|  |  |  |  | DPort |  | o | o |
|  |  |  |  | InterfaceList |  | o | o |
|  |  |  |  | Firewall |  | o | o |
|  |  |  |  | TTL |  | o | o |
|  |  |  | RoutingEnable |  |  | o | x |
|  |  |  | IsVlanSource |  |  | o | x |
|  |  |  | SelectedACLExtension |  |  | o | x |
|  |  |  | TTL |  |  | o | x |
|  |  |  | RoutingFirewall |  |  | o | x |
|  |  |  | DontCare |  |  | o | x |
|  |  |  | downgrade_L2 |  |  | o | x |
|  |  |  | EgrsMirror_Enbl |  |  | O | X |
|  |  |  | EgrsMirror_Enbl_P8 |  |  | O | X |
|  |  |  | EgrsMirror_Enbl_P9 |  |  | O | X |
|  |  |  | EgrsMirror_Enbl_txt |  |  | O | X |
|  |  |  | SelectedMirrorPort |  |  | O | X |
|  |  |  | SelectedSparePort |  |  | O | x |
|  |  |  | RoutingUniList | Index |  | o | x |
|  |  |  |  | Sip |  | o | x |
|  |  |  |  | Dip |  | o | x |
|  |  |  |  | IpType |  | o | x |
|  |  |  |  | SPort |  | o | x |
|  |  |  |  | DPort |  | o | x |
|  |  |  |  | L2Index |  | o | x |
|  |  |  |  | Firewall |  | o | x |
|  |  |  |  | EgressIdx |  | o | x |
|  |  |  |  | TTL |  | o | x |
|  |  |  | RoutingNetworkList | Sip |  | o | x |
|  |  |  |  | Dip |  | o | x |
|  |  |  |  | IpType |  | o | x |
|  |  |  |  | SPort |  | o | x |
|  |  |  |  | DPort |  | o | x |
|  |  |  |  | L2Index |  | o | x |
|  |  |  |  | SPortCare |  | o | x |
|  |  |  |  | DPortCare |  | o | x |
|  |  |  |  | EgressIdx |  | o | x |
|  |  |  | RoutingMultiList_image |  |  | o | x |
|  |  |  | RoutingUniList_image |  |  | o | x |
|  |  | GlobalAcl906x | EntryAclUdfList | Index |  | o | o |
|  |  |  |  | Type |  | o | o |
|  |  |  |  | Offset |  | o | o |
|  |  |  | EntryAclRangeList | Type |  | o | o |
|  |  |  |  | Index |  | o | o |
|  |  |  |  | SubType |  | o | o |
|  |  |  |  | UpperValue |  | o | o |
|  |  |  |  | LowerValue |  | o | o |
|  |  |  | EntryAclGroupList | Type |  | o | o |
|  |  |  |  | Index |  | o | o |
|  |  |  |  | SubType |  | o | o |
|  |  |  |  | Value |  | o | o |
|  |  |  | EntryAclAggreList | Index |  | o | x |
|  |  |  |  | AggOper |  | o | x |
|  |  |  | EntryAclList | ComboIndex |  | o | o |
|  |  |  |  | ComboField |  | o | o |
|  |  |  |  | Reverse |  | o | o |
|  |  |  |  | Action |  | o | o |
|  |  |  |  | Action1 |  | o | o |
|  |  |  |  | Action2 |  | o | o |
|  |  |  |  | Drop |  | o | o |
|  |  |  |  | Flow |  | o | o |
|  |  |  |  | Mirror |  | o | o |
|  |  |  |  | Copy |  | o | o |
|  |  |  |  | DPM |  | o | o |
|  |  |  |  | IsPolicerEnabled |  | o | o |
|  |  |  |  | Policer |  | o | o |
|  |  |  |  | AIF |  | o | o |
|  |  |  |  | AIF1 |  | o | o |
|  |  |  |  | AIF2 |  | o | o |
|  |  |  |  | Valid |  | o | o |
|  |  |  | 0 - 15 | SelectedPie |  | o | o |
|  |  |  |  | InputAIF |  | o | o |
|  |  |  |  | Policer |  | o | o |
|  |  |  |  | IsPolicerEnabled |  | o | o |
|  |  |  |  | InputPolicerVal |  | o | o |
|  |  |  |  | SelectedAction |  | o | o |
|  |  |  |  | DisplayedAction |  | o | o |
|  |  |  |  | SelectedFltr |  | o | o |
|  |  |  |  | DisplayedFltr |  | o | o |
|  |  |  |  | InputField |  | o | o |
|  |  |  |  | InputMask |  | o | o |
|  |  |  |  | InputAIF1 |  | o | x |
|  |  |  |  | InputAIF2 |  | o | x |
|  |  |  |  | SelectedAction1 |  | o | x |
|  |  |  |  | SelectedAction2 |  | o | x |
|  |  |  |  | DisplayedAction1 |  | o | x |
|  |  |  |  | DisplayedAction2 |  | o | x |
|  |  |  |  | SelectedDrop |  | o | x |
|  |  |  |  | SelectedFlow |  | o | x |
|  |  |  |  | SelectedMirror |  | o | x |
|  |  |  |  | SelectedCopy |  | o | x |
|  |  |  |  | InputCopyText |  | o | x |
|  |  |  | DefaultAction |  |  | o | x |
|  |  |  | ReserveAction |  |  | o | x |
|  |  | GlobalLut | MulticastHash |  |  | o | o |
|  |  |  | EntryLutList | Index |  | o | o |
|  |  |  |  | MemberPorts |  | o | o |
|  |  |  |  | Type |  | o | o |
|  |  |  |  | Mac |  | o | o |
|  |  |  |  | VId |  | o | o |
|  |  |  |  | Port |  | o | o |
|  |  |  |  | ULFId |  | o | o |
|  |  |  |  | FId |  | o | o |
|  |  |  |  | IsUnicast |  | o | o |
|  |  |  |  | IsMulticast |  | o | o |
|  |  | GlobalVlan | SPID |  |  | o | x |
|  |  |  | TPID |  |  | o | x |
|  |  |  | EntryVlanList | Vid |  | o | o |
|  |  |  |  | MemberPorts |  | o | o |
|  |  |  |  | UntaggedPorts |  | o | o |
|  |  |  |  | Fid |  | o | o |
|  |  |  |  | SharedVlan |  | o | o |
|  |  |  |  | VlanBasedFW |  | o | o |
|  |  |  |  | IsRemovable |  | o | o |
|  |  |  |  | IsFIdEnabled |  | o | o |
|  |  |  |  | VlanInterfaceId |  | o | o |
|  |  |  |  | VlanInterfaceValid |  | o | o |
|  |  | GlobalQos | PrioritySource |  |  | o | o |
|  |  |  | SelectedPrioritySource |  |  | o | o |
|  |  |  | SelectedQueueRemappingPort |  |  | o | o |
|  |  |  | PriDecQueueList | Item |  | o | o |
|  |  |  |  | Column0 |  | o | o |
|  |  |  |  | Column1 |  | o | o |
|  |  |  |  | Column2 |  | o | o |
|  |  |  |  | Column3 |  | o | o |
|  |  |  |  | Column4 |  | o | o |
|  |  |  |  | Column5 |  | o | o |
|  |  |  |  | Column6 |  | o | o |
|  |  |  |  | Column7 |  | o | o |
|  |  |  | PriDecInnerInnerList | Item |  | o | o |
|  |  |  |  | Column0 |  | o | o |
|  |  |  |  | Column1 |  | o | o |
|  |  |  |  | Column2 |  | o | o |
|  |  |  |  | Column3 |  | o | o |
|  |  |  |  | Column4 |  | o | o |
|  |  |  |  | Column5 |  | o | o |
|  |  |  |  | Column6 |  | o | o |
|  |  |  |  | Column7 |  | o | o |
|  |  |  | PriDecOuterOuterList | Item |  | o | o |
|  |  |  |  | Column0 |  | o | o |
|  |  |  |  | Column1 |  | o | o |
|  |  |  |  | Column2 |  | o | o |
|  |  |  |  | Column3 |  | o | o |
|  |  |  |  | Column4 |  | o | o |
|  |  |  |  | Column5 |  | o | o |
|  |  |  |  | Column6 |  | o | o |
|  |  |  |  | Column7 |  | o | o |
|  |  |  | PriDecDscpDscpList | Item |  | o | o |
|  |  |  |  | Column0 |  | o | o |
|  |  |  |  | Column1 |  | o | o |
|  |  |  |  | Column2 |  | o | o |
|  |  |  |  | Column3 |  | o | o |
|  |  |  |  | Column4 |  | o | o |
|  |  |  |  | Column5 |  | o | o |
|  |  |  |  | Column6 |  | o | o |
|  |  |  |  | Column7 |  | o | o |
|  |  |  | PriRmkInnerList | Item |  | o | o |
|  |  |  |  | Column0 |  | o | o |
|  |  |  |  | Column1 |  | o | o |
|  |  |  |  | Column2 |  | o | o |
|  |  |  |  | Column3 |  | o | o |
|  |  |  |  | Column4 |  | o | o |
|  |  |  |  | Column5 |  | o | o |
|  |  |  |  | Column6 |  | o | o |
|  |  |  |  | Column7 |  | o | o |
|  |  |  | PriRmkOuterList | Item |  | o | o |
|  |  |  |  | Column0 |  | o | o |
|  |  |  |  | Column1 |  | o | o |
|  |  |  |  | Column2 |  | o | o |
|  |  |  |  | Column3 |  | o | o |
|  |  |  |  | Column4 |  | o | o |
|  |  |  |  | Column5 |  | o | o |
|  |  |  |  | Column6 |  | o | o |
|  |  |  |  | Column7 |  | o | o |
|  |  |  | PriRmkDscpList | Item |  | o | o |
|  |  |  |  | Column0 |  | o | o |
|  |  |  |  | Column1 |  | o | o |
|  |  |  |  | Column2 |  | o | o |
|  |  |  |  | Column3 |  | o | o |
|  |  |  |  | Column4 |  | o | o |
|  |  |  |  | Column5 |  | o | o |
|  |  |  |  | Column6 |  | o | o |
|  |  |  |  | Column7 |  | o | o |
|  |  | GlobalFlowControl | FlowEnable |  |  | o | x |
|  |  |  | PortStatus |  |  | o | x |
|  |  | GlobalFrer | StreamStartIndex |  |  | o | x |
|  |  |  | StreamEndIndex |  |  | o | x |
|  |  |  | EntryStreamList | EntryIndex |  | o | x |
|  |  |  |  | Valid |  | o | x |
|  |  |  |  | DestAddr |  | o | x |
|  |  |  |  | SrcAddr |  | o | x |
|  |  |  |  | Ethertype |  | o | x |
|  |  |  |  | VlanID |  | o | x |
|  |  |  |  | SrcIP |  | o | x |
|  |  |  |  | DestIP |  | o | x |
|  |  |  |  | DSCP |  | o | x |
|  |  |  |  | NextProtocol |  | o | x |
|  |  |  |  | SrcPort |  | o | x |
|  |  |  |  | DestPort |  | o | x |
|  |  |  |  | PortMask |  | o | x |
|  |  |  |  | StreamHandle |  | o | x |
|  |  |  | CountersStartIndex |  |  | o | x |
|  |  |  | CountersEndIndex |  |  | o | x |
|  |  | GlobalPsfp | StreamFilterStartIndex |  |  | o | x |
|  |  |  | StreamFilterEndIndex |  |  | o | x |
|  |  |  | EntryStreamFilterList | EntryIndex |  | o | x |
|  |  |  |  | Valid |  | o | x |
|  |  |  |  | MatchingStreamHandle |  | o | x |
|  |  |  |  | MatchingPriority |  | o | x |
|  |  |  |  | StreamGateID |  | o | x |
|  |  |  |  | FlowMeterID |  | o | x |
|  |  |  |  | MaxSDUsize |  | o | x |
|  |  |  |  | BlockStreamEnable |  | o | x |
|  |  |  | StreamGateStartIndex |  |  | o | x |
|  |  |  | StreamGateEndIndex |  |  | o | x |
|  |  |  | EntryStreamGateList | EntryIndex |  | o | x |
|  |  |  |  | Valid |  | o | x |
|  |  |  |  | AdminGateState |  | o | x |
|  |  |  |  | AdminIPV |  | o | x |
|  |  |  |  | GateControlListEnable |  | o | x |
|  |  |  |  | GateClose_RX |  | o | x |
|  |  |  | EntryGclWindow | BaseTime_MsbSec |  | o | x |
|  |  |  | 0 - 95 | BaseTime_LsbSec |  | o | x |
|  |  |  |  | BaseTime_ns |  | o | x |
|  |  |  |  | CycleTimeEtn |  | o | x |
|  |  |  |  | SelectedGclLength |  | o | x |
|  |  |  |  | GateClose_Octet |  | o | x |
|  |  |  |  | EntryGclList | StreamGateIndex | o | x |
|  |  |  |  |  | ListIndex | o | x |
|  |  |  |  |  | GateState | o | x |
|  |  |  |  |  | IPV | o | x |
|  |  |  |  |  | Duration | o | x |
|  |  |  |  |  | IntervalOctets | o | x |
|  |  |  | FlowMeterStartIndex |  |  | o | x |
|  |  |  | FlowMeterEndIndex |  |  | o | x |
|  |  |  | EntryFlowMeterList | EntryIndex |  | o | x |
|  |  |  |  | Valid |  | o | x |
|  |  |  |  | CommitInfoRate |  | o | x |
|  |  |  |  | CommitBurstSize |  | o | x |
|  |  |  |  | ExcessInfoRate |  | o | x |
|  |  |  |  | ExcessBurstSize |  | o | x |
|  |  |  |  | CouplingFlag |  | o | x |
|  |  |  |  | ColorMode |  | o | x |
|  |  |  |  | DeiSelection |  | o | x |
|  |  |  |  | DropOnYellowFrame |  | o | x |
|  |  |  |  | IPVforYellowFrame |  | o | x |
|  |  |  |  | MarkAllFramesRed |  | o | x |
|  |  |  | CountersStartIndex |  |  | o | x |
|  |  |  | CountersEndIndex |  |  | o | x |
|  |  |  | ErrorStartIndex |  |  | o | x |
|  |  |  | ErrorEndIndex |  |  | o | x |
|  |  | GlobalPhyMaster | InputPhyID |  |  | o | x |
|  |  |  | SelectPower_Rate |  |  | o | x |
|  |  |  | SelectedPhyIdMode |  |  | o | x |
|  |  |  | PhyAddr |  |  | o | x |
|  |  |  | PhyVal |  |  | o | x |
|  |  |  | SelectedMappingMode |  |  | o | x |
|  |  |  | Mapping_Page |  |  | o | x |
|  |  |  | Mapping_Reg |  |  | o | x |
|  |  |  | Mapping_Ext |  |  | o | x |
|  | Port | 0 - 14 | PortGeneral | SelectedPortIf |  | o | o |
|  | Port_S1 |  |  | Role |  | o | o |
|  | Port_S2 |  |  | PhyStatus |  | o | o |
|  | Port_S3 |  |  | IsWUPEnabled |  | x | o |
|  |  |  |  | PhyPower |  | x | o |
|  |  |  |  | isForce |  | o | o |
|  |  |  |  | isSerdes |  | o | o |
|  |  |  |  | SelectedLinkSpeed |  | o | o |
|  |  |  |  | RxDelay |  | o | o |
|  |  |  |  | TxDelay |  | o | o |
|  |  |  |  | SelectedLdoVoltage |  | o | o |
|  |  |  |  | SelectedRGMIIVoltage |  | o | o |
|  |  |  |  | SelectedSpeed |  | o | o |
|  |  |  |  | RGMIIDriving |  | o | o |
|  |  |  |  | RGMIIDrivingLow |  | o | o |
|  |  |  |  | DrivingEnable |  | o | o |
|  |  |  |  | SelectedE0PortIf |  | x | o |
|  |  |  |  | SelectedSgmiiExternalPortCfg |  | o | o |
|  |  |  |  | SelectedRgmiiTxDelay |  | x | o |
|  |  |  |  | SelectedRgmiiRxDelay |  | x | o |
|  |  |  |  | Rolelabel |  | o | x |
|  |  |  |  | DrivingEnablelabel |  | o | x |
|  |  |  |  | isSerdeslabel |  | o | x |
|  |  |  |  | isForcelabel |  | o | x |
|  |  |  | PortVlan | PortNum |  | o | o |
|  |  |  |  | InputVId |  | o | o |
|  |  |  |  | InputPriority |  | o | o |
|  |  |  |  | SelectedLearningBased |  | o | o |
|  |  |  |  | SelectedMode |  | o | o |
|  |  |  |  | IngressFilter |  | o | o |
|  |  |  |  | EgressFilter |  | o | o |
|  |  |  |  | SelectedFrameType |  | o | o |
|  |  |  |  | DisplayedFrameType |  | o | o |
|  |  |  | PortQos | PriDecPriority |  | o | o |
|  |  |  |  | PriRmkInnerTag |  | o | o |
|  |  |  |  | PriRmkOuterTag |  | o | o |
|  |  |  |  | PriRmkDscp |  | o | o |
|  |  |  |  | PriRmkBoundaryPort |  | o | o |
|  |  |  |  | PriRmkBoundaryPortList | Item | o | o |
|  |  |  |  |  | Column0 | o | o |
|  |  |  |  |  | Column1 | o | o |
|  |  |  |  |  | Column2 | o | o |
|  |  |  |  |  | Column3 | o | o |
|  |  |  |  |  | Column4 | o | o |
|  |  |  |  |  | Column5 | o | o |
|  |  |  |  |  | Column6 | o | o |
|  |  |  |  |  | Column7 | o | o |
|  |  |  |  | SelectedQueueSchedComb |  | o | o |
|  |  |  |  | DisplayedSchedItem0 |  | o | o |
|  |  |  |  | DisplayedSchedItem1 |  | o | o |
|  |  |  |  | QueueSchedItem0 |  | o | o |
|  |  |  |  | QueueSchedItem1 |  | o | o |
|  |  |  |  | SelectedQueueSched |  | o | o |
|  |  |  |  | IsQueueWeightEnabled |  | o | o |
|  |  |  |  | IsRateLimitingEnabled |  | o | o |
|  |  |  |  | IsRateEnableOn |  | o | o |
|  |  |  |  | InputQueueWeight |  | o | o |
|  |  |  |  | InputRateLimiting |  | o | o |
|  |  |  |  | IsCbsBwEnabled |  | o | o |
|  |  |  |  | InputCbsBw |  | o | o |
|  |  |  | PortTraffic | AdminGateState |  | o | x |
|  |  |  |  | OperGateState |  | o | x |
|  |  |  |  | GateEnableState |  | o | x |
|  |  |  |  | QueueMaxSDU |  | o | x |
|  |  |  |  | BaseTime_MsbSec |  | o | x |
|  |  |  |  | BaseTime_LsbSec |  | o | x |
|  |  |  |  | BaseTime_ns |  | o | x |
|  |  |  |  | CycleTimeEtn |  | o | x |
|  |  |  |  | ControlListLength |  | o | x |
|  |  |  |  | EntryTrafficList | Index | o | x |
|  |  |  |  |  | GateState | o | x |
|  |  |  |  |  | Interval | o | x |
|  |  |  |  | CycleTime |  | o | x |
|  |  |  |  | SelectedPortType |  | o | x |
|  |  |  | PortStormControl | BroadcastStormControl |  | o | x |
|  |  |  |  | BcDataRateType |  | o | x |
|  |  |  |  | BroadcastDataRate |  | o | x |
|  |  |  |  | MulticastStormControl |  | o | x |
|  |  |  |  | McDataRateType |  | o | x |
|  |  |  |  | MulticastDataRate |  | o | x |
|  |  |  |  | SelectedMulticastType |  | o | x |
|  |  |  |  | UnicastStormControl |  | o | x |
|  |  |  |  | UcDataRateType |  | o | x |
|  |  |  |  | UnicastDataRate |  | o | x |
|  |  |  | PortIsolation | BlockedFwPortMask |  | o | x |

Global Configuration

TabGeneral

Tool: General

Example:

"TabGeneral": {

"SavedChip": "RTL9075AVA",

"SavedIF": "I2C",

"ConfigOutputPath": "",

"EtherDis": true,

"JSON_Version": "2.1.6.1.0"

}

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| SavedChip | String | "Chip Version" | "RTL9043AA" "RTL9044AA" "RTL9044AB" "RTL9047AA" "RTL9072AV2" "RTL9068AV2" "RTL9075AV2" "RTL9072AV3" "RTL9068AV3" "RTL9075AV3" "RTL9075AVA" "RTL9072AVA" "RTL9068AVA" "RTL9054AVA" | "RTL9043AA" "RTL9075VA" | R/W |
| SavedIF | String | “Configuration Mode” | “I2C” “BINARY” “MDC_MDIO” “SPI” “ETHERNET” | “I2C” | R/W |
| EtherDis | Boolean | "Enabled/Disabled" | true or false | true | RO |

TabStacking

Tool: Stacking

Example:

"TabStacking": {

"GroupID": "1",

"EntryStackingList": [

{

"Model": "RTL9075AVA",

"MAC Address": "00:00:00:00:00:00",

"isMaster": "False",

"Unit ID": "1",

"S0 Link To": "OFF",

"S1 Link To": "OFF"

}

]

}

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| GroupID | String | "Group ID" | Register address in 16 bits hex string format (EX. "0x5566") | "1" | R/W |
| Model | String | "Model" | "RTL9072AV2" "RTL9068AV2" "RTL9075AV2" "RTL9072AV3" "RTL9068AV3" "RTL9075AV3" "RTL9075AVA" "RTL9072AVA" "RTL9068AVA" "RTL9054AVA" | "RTL9075AVA" | R/W |
| MAC Address | String | "MAC Address" | MAC address in string format | "00:00:00:00:00:00" | R/W |
| isMaster | String | "Master" | "True" or "False" | "False" | R/W |
| Unit ID | String | "Unit ID" | "1"-"7" | "1" | R/W |
| S0 Link To | String | "S0 Link To" | "1"-"7", "OFF" and "N/A" | "OFF" | R/W |
| S1 Link To | String | "S1 Link To" | "1"-"7", "OFF" and "N/A" | "OFF" | R/W |

GlobalPtp9075

Tool: Switch -> Global Settings -> PTP

Example:

"GlobalPtp9075": {

"Slave0JoinedPtpPorts": [

false, false, false, false, false,

false, false, false, false, false,

false, false, false, false, false

],

"Slave1JoinedPtpPorts": [

false, false, false, false, false,

false, false, false, false, false,

false, false, false, false, false

],

"Slave2JoinedPtpPorts": [

false, false, false, false, false,

false, false, false, false, false,

false, false, false, false, false

],

"SelectedEnableSync": "Disable",

"SelectedBasicMode": "P2P",

"SelectedSlave0": "Port4",

"SelectedSlave1": "Port4",

"SelectedSlave2": "Port4",

"Slave0Domain": 0,

"Slave1Domain": 1,

"Slave2Domain": 2,

"Slave0Pri": 248,

"Slave1Pri": 248,

"Slave2Pri": 248,

"SyncInterval": "1s",

"pdelayReq": "1s",

"SelectedGPIOB1": "Local",

"SelectedGPIOB2": "Local",

"GPIOB1": false,

"GPIOB2": false,

"SendPdelayReq": true,

"AssignPriority": false,

"AsCapableMaskDef": "0x7FFF",

"DelayTh_Sec": 0,

"DelayTh_NanoSec": 524296000,

"PortNumRemappingEnable": false,

"PortNumRemapping": [

0, 1, 2, 3, 4,

5, 6, 7, 8, 9,

10, 11, 12, 13, 14

],

"OneStepTx0": false,

"OneStepTx1": false,

"OneStepTx2": false,

"SignalEnableDomain0": false,

"SignalEnableDomain1": false,

"SignalEnableDomain2": false,

"VlanSupport0": false,

"VlanSupport1": false,

"VlanSupport2": false,

"VlanSupportVID0": 0,

"VlanSupportVID1": 0,

"VlanSupportVID2": 0,

"PacketDestAddrEnable": false,

"PacketDestAddr": "01:80:C2:00:00:0E",

"PacketEtherTypeEnable": false,

"PacketEtherType": "0x88F7",

"NoSyncRecovery": false,

"OverwriteClockID": false,

"TimeoutVal": 20000

}

| Key | Type | Items On Tool |  | Accepted Value |  | Default | R/W |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Slave0JoinedPtpPorts | Array(Boolean) | In "Basic" section: "Joined Port" of "Slave 0" |  | Array of 15 Booleans |  | Array of 15 false | R/W |
| Slave1JoinedPtpPorts | Array(Boolean) | In "Basic" section: "Joined Port" of "Slave 1" |  | Array of 15 Booleans |  | Array of 15 false | R/W |
| Slave2JoinedPtpPorts | Array(Boolean) | In "Basic" section: "Joined Port" of "Slave 2" |  | Array of 15 Booleans |  | Array of 15 false | R/W |
| SelectedEnableSync | String | In "Basic" section: "Time Sync" |  | "Enable" or "Disable" |  | "Disable" | R/W |
| SelectedBasicMode | String | In "Basic" section: "Mode" |  | "P2P TC+BC" or "Grandmaster" |  | "P2P TC+BC" | R/W |
| SelectedSlave0 | String | In "Basic" section: "Slave Port" of "Slave 0" |  | "Port0", "Port1", ..., "Port14" |  | "Port4" | R/W |
| SelectedSlave1 | String | In "Basic" section: "Slave Port" of "Slave 1" |  | "Port0", "Port1", ..., "Port14" |  | "Port4" | R/W |
| SelectedSlave2 | String | In "Basic" section: "Slave Port" of "Slave 2" |  | "Port0", "Port1", ..., "Port14" |  | "Port4" | R/W |
| Slave0Domain | Number | Internal use only for debugging |  |  |  |  | RO |
| Slave1Domain | Number | Internal use only for debugging |  |  |  |  | RO |
| Slave2Domain | Number | Internal use only for debugging |  |  |  |  | RO |
| Slave0Pri | Number | Internal use only for debugging |  |  |  |  | RO |
| Slave1Pri | Number | Internal use only for debugging |  |  |  |  | RO |
| Slave2Pri | Number | Internal use only for debugging |  |  |  |  | RO |
| SyncInterval | String | In "Interval" section: "Sync Interval" |  | 2^(-5) - 2^3 s |  | "1/8s" | R/W |
| pdelayReq | String | In "Interval" section: "Pdelay Request" |  | 2^(-5) - 2^3 s |  | "1s" | R/W |
| SelectedGPIOB1 | String | In "PPS" section: "Clock Selection" of "GPIOB1" |  | "Local", "Master" "Slave 0", "Slave 1" and "Slave 2" |  | "Local" | R/W |
| SelectedGPIOB2 | String | In "PPS" section: "Clock Selection" of "GPIOB2" |  | "Local", "Master", "Slave 0", "Slave 1" and "Slave 2" |  | "Local" | R/W |
| GPIOB1 | Boolean | In "PPS" section: "Enable" of "GPIOB1" |  | true or false |  | false | R/W |
| GPIOB2 | Boolean | In "PPS" section: "Enable" of "GPIOB2" |  | true or false |  | false | R/W |
| SendPdelayReq | Boolean | Internal use only for debugging |  |  |  |  | RO |
| AssignPriority | Boolean | Internal use only for debugging |  |  |  |  | RO |
| AsCapableMaskDef | Number | Internal use only for debugging |  |  |  |  | RO |
| DelayTh_Sec | Number | Internal use only for debugging |  |  |  |  | RO |
| DelayTh_NanoSec | Number | Internal use only for debugging |  |  |  |  | RO |
| PortNumRemappingEnable | Boolean | Internal use only for debugging |  |  |  |  | RO |
| PortNumRemapping | Array(Boolean) | Internal use only for debugging |  |  |  |  | RO |
| OneStepTx0 | Boolean | Internal use only for debugging |  |  |  |  | RO |
| OneStepTx1 | Boolean | Internal use only for debugging |  |  |  |  | RO |
| OneStepTx2 | Boolean | Internal use only for debugging |  |  |  |  | RO |
| SignalEnableDomain0 | Boolean | Internal use only for debugging |  |  |  |  | RO |
| SignalEnableDomain1 | Boolean | Internal use only for debugging |  |  |  |  | RO |
| SignalEnableDomain2 | Boolean | Internal use only for debugging |  |  |  |  | RO |
| VlanSupport0 | Boolean | Internal use only for debugging |  |  |  |  | RO |
| VlanSupport1 | Boolean | Internal use only for debugging |  |  |  |  | RO |
| VlanSupport2 | Boolean | Internal use only for debugging |  |  |  |  | RO |
| VlanSupportVID0 | Number | Internal use only for debugging |  |  |  |  | RO |
| VlanSupportVID1 | Number | Internal use only for debugging |  |  |  |  | RO |
| VlanSupportVID2 | Number | Internal use only for debugging |  |  |  |  | RO |
| PacketDestAddrEnable | Boolean | Internal use only for debugging |  |  |  |  | RO |
| PacketDestAddr | String | Internal use only for debugging |  |  |  |  | RO |
| PacketEtherTypeEnable | Boolean | Internal use only for debugging |  |  |  |  | RO |
| PacketEtherType | String | Internal use only for debugging |  |  |  |  | RO |
| NoSyncRecovery | Boolean | Internal use only for debugging |  |  |  |  | RO |
| OverwriteClockID | Boolean | Internal use only for debugging |  |  |  |  | RO |
| TimeoutVal | Number | In "MISC" section: TC+BC Timeout value | Integer |  | 20000 |  | R/W |

GlobalMirror906x

Tool: Switch -> Global Settings -> Mirror

Example:

"GlobalMirror906x": {

"MirroredRxPorts": [

false, false, false, false, false,

false, false, false, false, false,

false, false, false, false, false

],

"MirroredTxPorts": [

false, false, false, false, false,

false, false, false, false, false,

false, false, false, false, false

],

"BlockedFwPortMask": [

false, false, false, false, false,

false, false, false, false, false,

false, false, false, false, false

],

"SelectedPortBasedMirrorNo": 1,

"SelectedPortBasedMirrorPort": 0,

"SelectedMirrorNo": 1,

"IsMirrorFilterOn": false,

"SelectedPortBasedMirrorPort_1": 0,

"MirroredRxPorts_1": [

false, false, false, false, false,

false, false, false, false, false,

false, false, false, false, false

],

"MirroredTxPorts_1": [

false, false, false, false, false,

false, false, false, false, false,

false, false, false, false, false

],

"IsMirrorFilterOn_1": "Disabled",

"InputSampleRate_1": "1",

"SelectedRSPAN_Action_1": "Retain",

"InputTPID_1": "0x8810",

"InputPCP_1": "0",

"InputCFI_1": "0",

"InputTagVID_1": "0",

"IsRSPAN_Check_1": false,

"InputCheckVID_1": "0",

"SelectedPortBasedMirrorPort_2": 0,

"MirroredRxPorts_2": [

false, false, false, false, false,

false, false, false, false, false,

false, false, false, false, false

],

"MirroredTxPorts_2": [

false, false, false, false, false,

false, false, false, false, false,

false, false, false, false, false

],

"IsMirrorFilterOn_2": "Disabled",

"InputSampleRate_2": "1",

"SelectedRSPAN_Action_2": "Retain",

"InputTPID_2": "0x8810",

"InputPCP_2": "0",

"InputCFI_2": "0",

"InputTagVID_2": "0",

"IsRSPAN_Check_2": false,

"InputCheckVID_2": "0",

"IsMirrorTopologyEnable_1": false,

"EgressPortsEnable_1": true,

"MirroringTopologySet_1": "Local Mirroring",

"IsMirrorTopologyEnable_2": false,

"EgressPortsEnable_2": true,

"MirroringTopologySet_2": "Local Mirroring",

"MirrorTopologyEnable_1": false,

"MirrorTopologyEnable_2": false

}

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| MirroredRxPorts | Array(Boolean) | “Mirrored Rx Ports Mask” | Array of 15 Booleans | Array of 15 false | R/W |
| MirroredTxPorts | Array(Boolean) | “Mirrored Tx Ports Mask” | Array of 15 Booleans | Array of 15 false | R/W |
| BlockedFwPortMask | Array(Boolean) | “Blocked Forwarding Ports” | Array of 15 Booleans | Array of 15 false | R/W |
| SelectedPortBasedMirrorNo | Number | "Function 1" and "Function 2" | 1 or 2 | 1 | R/W |
| SelectedPortBasedMirrorPort | Number | “Mirroring Port” | 0-14 | 0 | R/W |
| SelectedMirrorNo | Number | In "Mirror Filter On/Off" section: "Function 1" or "Function 2" | 1 or 2 | 1 | R/W |
| IsMirrorFilterOn | Boolean | In "Mirror Filter On/Off" section: "Enabled" or "Disabled" | true or false | false | R/W |
| SelectedPortBasedMirrorPort_1 SelectedPortBasedMirrorPort_2 | Number | “Mirroring Port” | 0-14 | 0 | R/W |
| MirroredRxPorts_1 MirroredRxPorts_2 | Array(Boolean) | "Ingress Ports" | Array of 15 Booleans | Array of 15 false | R/W |
| MirroredTxPorts_1 MirroredTxPorts_2 | Array(Boolean) | "Egress Ports" | Array of 15 Booleans | Array of 15 false | R/W |
| IsMirrorFilterOn_1 IsMirrorFilterOn_2 | Boolean | In "Mirror Filter On/Off" section: "Enabled" or "Disabled" | true or false | false | R/W |
| InputSampleRate_1 InputSampleRate_2 | Number | "Sample Rate" | Integer | 1 | R/W |
| SelectedRSPAN_Action_1 SelectedRSPAN_Action_2 | String | "RSPAN Tag" | "Retain" or "Add" | "Retain" | R/W |
| InputTPID_1 InputTPID_2 | String | "TPID Field" | Hex string format | "0x8810" | R/W |
| InputPCP_1 InputPCP_2 | Number | "PCP Field" | Integer | 0 | R/W |
| InputCFI_1 InputCFI_2 | Number | "CFI Field" | Integer | 0 | R/W |
| InputTagVID_1 InputTagVID_2 | Number | "VID Field" | Integer | 0 | R/W |
| IsRSPAN_Check_1 IsRSPAN_Check_2 | Boolean | In "Check RSPAN Tag" section: "Enabled" or "Disabled" | true or false | false | R/W |
| InputCheckVID_1 InputCheckVID_2 | Number | "VLAN Identifier (VID)" | Integer | 0 | R/W |
| IsMirrorTopologyEnable_1 IsMirrorTopologyEnable_2 | Boolean | Internal use only for debugging |  |  | RO |
| EgressPortsEnable_1 EgressPortsEnable_2 | Boolean | Internal use only for debugging |  |  | RO |
| MirroringTopologySet_1 MirroringTopologySet_2 | String | "Mirroring Topology" | "Disable", "Local Mirroring" "Remote Mirroring" | "Local Mirroring" | R/W |
| MirrorTopologyEnable_1 MirrorTopologyEnable_2 | Boolean | Internal use only for debugging |  |  | RO |
| MirroringTopologyTxt_1 MirroringTopologyTxt_2 | String | Internal use only for debugging |  |  | RO |

GlobalMisc906x

Tool: Switch -> Global Settings -> Misc (Runtime mode only)

Example:

"GlobalMisc906x": {

"PhyId": 0,

"TestMode": 1,

"Is1000BaseT1": false,

"Is1000BaseT1Master": false

}

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| PhyId | Number | “Port Id” | 0,1,2,3,4,5,6,7,9,10 | 0 | R/W |
| TestMode | Number | “Test Mode” | 1,2,3,4,5,6 | 1 | R/W |
| Is1000BaseT1 | Boolean | “100/1000Base-T1” | true or false | false | R/W |
| Is1000BaseT1Master | Boolean | “Master/Slave” | true or false | false | R/W |

GlobalSleep

Tool: Switch -> Global Settings -> Sleep

Example:

"GlobalSleep": {

"SelectedSleepWakeConfig": "TC10 Sleep Capability",

"WholeChipSleep": false,

"SelectedOPLocalState": "Sleep",

"SelectedOPGlobalState": "Sleep",

"SelectedRemoteWake": "Disable",

"SelectedRemotWakePorts": "0",

"SleepWakePorts": [

false, false, false, false, false,

false, false, false, false, false,

false

],

"OPLocalStatePorts": [

false, false, false, false, false,

false, false, false, false, false,

false

],

"RemoteWakePorts": [

false, false, false, false, false,

false, false, false, false, false,

false

],

"TC10SC_Ports": [

false, false, false, false, false,

false, false, false, false, false,

false

],

"AcceptLP_Ports": [

false, false, false, false, false,

false, false, false, false, false,

false

],

"ForwardSR_Ports": [

false, false, false, false, false,

false, false, false, false, false,

false

],

"WholeChipS_Ports": [

false, false, false, false, false,

false, false, false, false, false,

false

],

"ForwardLP_Ports": [

false, false, false, false, false,

false, false, false, false, false,

false

],

"DisableRWD_Ports": [

false, false, false, false, false,

false, false, false, false, false,

false

],

"RemoteWakeEnable": [

0, 0, 0, 0, 0,

0, 0, 0, 0, 0,

0

],

"RemoteWakePorts_0": [

false, false, false, false, false,

false, false, false, false, false,

false

],

"RemoteWakePorts_1": [

false, false, false, false, false,

false, false, false, false, false,

false

],

"RemoteWakePorts_2": [

false, false, false, false, false,

false, false, false, false, false,

false

],

"RemoteWakePorts_3": [

false, false, false, false, false,

false, false, false, false, false,

false

],

"RemoteWakePorts_4": [

false, false, false, false, false,

false, false, false, false, false,

false

],

"RemoteWakePorts_5": [

false, false, false, false, false,

false, false, false, false, false,

false

],

"RemoteWakePorts_6": [

false, false, false, false, false,

false, false, false, false, false,

false

],

"RemoteWakePorts_7": [

false, false, false, false, false,

false, false, false, false, false,

false

],

"RemoteWakePorts_8": [

false, false, false, false, false,

false, false, false, false, false,

false

],

"RemoteWakePorts_9": [

false, false, false, false, false,

false, false, false, false, false,

false

],

"RemoteWakePorts_10": [

false, false, false, false, false,

false, false, false, false, false,

false

]

}

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| SelectedSleepWakeConfig | String | "Configure Setting" | "TC10 Sleep Capability", etc. | "TC10 Sleep Capability" | R/W |
| WholeChipSleep | Boolean | "Whole Chip Sleep" | true or false | false | R/W |
| SelectedOPLocalState | String | "Set Local State" | "Sleep", etc. | "Sleep" | R/W |
| SelectedOPGlobalState | String | "Set Global State" | "Sleep", etc. | "Sleep" | R/W |
| SelectedRemoteWake | Boolean | "Remote Wake Up" | "Disable" or "Enable" | "Disable" | R/W |
| SelectedRemotWakePorts | Number | "Ports" | 0-10 | 0 | R/W |
| SleepWakePorts | Array (Boolean) | "Configure Ports" | Array of 11 Booleans | Array of 11 false | R/W |
| OPLocalStatePorts | Array (Boolean) | "Local Ports" | Array of 11 Booleans | Array of 11 false | R/W |
| RemoteWakePorts | Array (Boolean) | "Remote Ports" | Array of 11 Booleans | Array of 11 false | R/W |
| TC10SC_Ports | Array (Boolean) | "TC10 SC Ports" | Array of 11 Booleans | Array of 11 false | R/W |
| AcceptLP_Ports | Array (Boolean) | "Accept LP Ports" | Array of 11 Booleans | Array of 11 false | R/W |
| ForwardSR_Ports | Array (Boolean) | "Forward SR Ports" | Array of 11 Booleans | Array of 11 false | R/W |
| WholeChipS_Ports | Array (Boolean) | "Whole Chip S Ports" | Array of 11 Booleans | Array of 11 false | R/W |
| ForwardLP_Ports | Array (Boolean) | "Forward LP Ports" | Array of 11 Booleans | Array of 11 false | R/W |
| DisableRWD_Ports | Array (Boolean) | "Disable RWD Ports" | Array of 11 Booleans | Array of 11 false | R/W |
| RemoteWakeEnable | Array (Boolean) | "Remote Wake Enable" | Array of 11 Booleans | Array of 11 false | R/W |
| RemoteWakePorts_0 | Array (Boolean) | "Remote Wake Ports 0" | Array of 11 Booleans | Array of 11 false | R/W |
| RemoteWakePorts_1 | Array (Boolean) | "Remote Wake Ports 1" | Array of 11 Booleans | Array of 11 false | R/W |
| RemoteWakePorts_2 | Array (Boolean) | "Remote Wake Ports 2" | Array of 11 Booleans | Array of 11 false | R/W |
| RemoteWakePorts_3 | Array (Boolean) | "Remote Wake Ports 3" | Array of 11 Booleans | Array of 11 false | R/W |
| RemoteWakePorts_4 | Array (Boolean) | "Remote Wake Ports 4" | Array of 11 Booleans | Array of 11 false | R/W |
| RemoteWakePorts_5 | Array (Boolean) | "Remote Wake Ports 5" | Array of 11 Booleans | Array of 11 false | R/W |
| RemoteWakePorts_6 | Array (Boolean) | "Remote Wake Ports 6" | Array of 11 Booleans | Array of 11 false | R/W |
| RemoteWakePorts_7 | Array (Boolean) | "Remote Wake Ports 7" | Array of 11 Booleans | Array of 11 false | R/W |
| RemoteWakePorts_8 | Array (Boolean) | "Remote Wake Ports 8" | Array of 11 Booleans | Array of 11 false | R/W |
| RemoteWakePorts_9 | Array (Boolean) | "Remote Wake Ports 9" | Array of 11 Booleans | Array of 11 false | R/W |
| RemoteWakePorts_10 | Array (Boolean) | "Remote Wake Ports 10" | Array of 11 Booleans | Array of 11 false | R/W |

GlobalReg

Tool: Switch -> Global Settings -> Register (Binary mode only)

Example:

"GlobalReg": {

"AddRegType": "MAC",

"EntryRegList": [

{

"RegAddr": "0x18000C",

"DefaultRegVal": "0x00000000",

"ConfigRegVal": "0x841",

"RegPage": null,

"SgType": "N_A",

"IsPageEnabled": false,

"EnabledPorts": null,

"Addr_Len": 6,

"DefRegVal_Len": 8,

"CfgRegVal_Len": 10,

"RegType": "MAC"

},

{

"RegAddr": "0x000000",

"DefaultRegVal": "0x00000000",

"ConfigRegVal": "0x00000000",

"RegPage": null,

"SgType": "N_A",

"IsPageEnabled": false,

"EnabledPorts": null,

"Addr_Len": 8,

"DefRegVal_Len": 8,

"CfgRegVal_Len": 10,

"RegType": "MAC"

}

]

}

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| AddRegType | String | "Reg Type" | "MAC", "SGMII", "FWCFG", "RTPG_PHY_1" and "RTPG_PHY_2" | "MAC" | R/W |
| EntryRegList | Array (JSON Object) | Register Table | JSON object (Containing RegAddr, DefaultRegVal, ConfigRegVal, RegType) | Empty array | R/W |
| RegAddr | String | “Register Address” | Register address in 24 bits hex string format (EX. “0x180000”) | “0x000000” | R/W |
| DefaultRegVal | String | “Default Register Value” | Register value in 32 bits hex string format (EX. “0x00000841” or “0x841”) | “0x00000000” | R/W |
| ConfigRegVal | String | “Config Register Value” | Register value in 32 bits hex string format (EX. “0x00000843” or “0x843”) | “0x00000000” | R/W |
| RegPage | Number | "Page" | Integer | null | R/W |
| SgType | String | "SG Type" | "N_A", "D_PHY" and "PCS" | "N_A" | R/W |
| IsPageEnabled | Boolean | Internal use only for debugging |  |  | RO |
| EnabledPorts | String | "Port enabled" | Port string array, for example, 6, 7, 8-11 | null | R/W |
| Addr_Len | Number | Internal use only for debugging |  |  | RO |
| DefRegVal_Len | Number | Internal use only for debugging |  |  | RO |
| CfgRegVal_Len | Number | Internal use only for debugging |  |  | RO |
| RegType | String | “Register Type” | "MAC", "SGMII", "FWCFG", "RTPG_PHY_1" and "RTPG_PHY_2" | “MAC” | R/W |

GlobalAcl906x

Tool: Switch -> Global Settings -> ACL

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| EntryAclUdfList | Array (JSON Object) | ACL Pie Table | JSON object (Index, Type, Offset) | empty array | R/W |
| EntryAclRangeList | Array (JSON Object) | ACL Pie Table | JSON object (Type, Index, SubType, UpperValue, LowerValue) | empty array | R/W |
| EntryAclGroupList | Array (JSON Object) | ACL Pie Table | JSON object (Type, Index, SubType, Value) | empty array | R/W |
| EntryAclList | Array (JSON Object) | ACL Pie Table | JSON object (Containing ComboIndex, ComboField, Action, IsPolicerEnabled, AIF, Valid) | empty array | R/W |
| Template Generator “0” – “7” | JSON Object | “Template” | JSON object (Containing SelectedPie, InputAIF, Policer, IsPolicerEnabled, InputPolicerVal, SelectedAction, DisplayedAction, SelectedFltr) | JSON Object with default attribute values | R/W |

EntryAclUdfList

Example:

"GlobalAcl906x": {

"EntryAclUdfList": [

{

"Index": 0,

"Type": "IPv4",

"Offset": 0,

}

]

}

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| Index | Number | User Defined Field -> Index | 0-31 | 0 | R/W |
| Type | String | User Defined Field -> Type | "IPv4", "IPv6", "L3", "TCP", "UDP", "L4", "Payload" or "Max" | "IPv4" | R/W |
| Offset | Number | User Defined Field -> Offset | 0-1023 | 0 | R/W |

EntryAclRangeList

Example:

"GlobalAcl906x": {

"EntryAclRangeList": [

{

"Type": "IPv4",

"Index": 0,

"SubType": "SIP",

"UpperValue": "0.0.0.0",

"LowerValue": "0.0.0.0",

}

]

}

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| Type | String | Range Check -> Type | "IPv4", "IPv6", "L4 Port", "VLAN ID", "Packet Length" | "IPv4" | R/W |
| Index | Number | Range Check -> Index | 0-31 | 0 | R/W |
| SubType | String | Range Check -> SubType | "SIP", "DIP", "SIP or DIP" or "Invalid" | "SIP" | R/W |
| UpperValue | String | Range Check -> UpperValue | Value depending on Key "Type" | No default value | R/W |
| LowerValue | String | Range Check -> LowerValue | Value depending on Key "Type" | No default value | R/W |

EntryAclGroupList

Example:

"GlobalAcl906x": {

"EntryAclGroupList": [

{

"Type": "IPv4",

"Index": 0,

"SubType": "SIP",

"Value": "0.0.0.0",

}

]

}

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| Type | String | Group Member Check -> Type | "IPv4", "IPv6", "L4 Port", "VLAN ID" | "IPv4" | R/W |
| Index | Number | Group Member Check -> Index | 0-31 | 0 | R/W |
| SubType | String | Group Member Check -> SubType | "SIP" or "DIP" | "SIP" | R/W |
| Value | String | Group Member Check -> Value | Value depending on Key "Type" | No default value | R/W |

EntryAclAggreList

Example:

"GlobalAcl906x": {

"EntryAclGroupList": [

{

"Index": 0,

"AggOper": 0

}

]

}

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| Index | Number | Group Member Check -> Index | 0-511 | 0 | R/W |
| AggOper | Number | Group Member Check -> AggOper | 0-2 | 0 | R/W |

EntryAclList

Example:

"GlobalAcl906x": {

"EntryAclList": [

{

"ComboIndex": "74",

"ComboField": [

"1122/ffff", "3344/ffff", "0000/0000", "0000/0000", "0000/0000",

"0000/0000", "0000/0000", "0000/0000", "0000/0000", "0000/0000",

"0000/0000", "0000/0000"

],

"Reverse": "Off",

"Action": "Forward",

"Action1": "Action",

"Action2": "Action",

"Drop": "Bypass",

"Flow": "Flow 0",

"Mirror": "None",

"Copy": "None",

"DPM": "0",

"IsPolicerEnabled": true,

"Policer": "01:1000",

"AIF": "00000000",

"AIF1": "00000000",

"AIF2": "00000000",

"Valid": "1"

}

]

}

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| ComboIndex | String | “PIE Entry” | “0” – “511” | No default value | R/W |
| ComboField | Array (String) | “0” - “11” | Combination of field value and mask in String format. (EX. “8899/ffff”) | “0000/0000” | R/W |
| Action, Action1, Action2 | String | “Action0”, “Action1”, “Action2” | "Action", "Forward", "OVID Remark", "IVID Remark", "PRI Remark", "DSCP Remark" and "Priority" | No default value | R/W |
| Reverse | String | "Hit Result Reverse" | “On”, “Off” | “Off” | R/W |
| Drop | String | “Drop Action” | “Bypass”, “Drop”, “Withdraw Drop” | “Bypass” | R/W |
| Flow | String | “Flow Action” | “Flow 0”, “Flow 1” | “Flow 0” | R/W |
| Mirror | String | “Mirror Action” | “None”, “Mirror 0”, “Mirror 1” | “None” | R/W |
| Copy | String | “Copy Action” | “Copy2CPU”, “None” | “None” | R/W |
| DPM | String | "DPM" | Destination port mask | "0" | R/W |
| IsPolicerEnabled | Boolean | Internal use only for debugging |  |  | RO |
| Policer | String | “Policer” | Combination of policer parameters: “01”(policer enabled) or “00”(disabled) + “:” + Policer value in String format. EX. “01:1000”, “00:0000” | “00:0000” | R/W |
| AIF, AIF1, AIF2 | String | “AIF”, “AIF1”, “AIF2” | Action field value in 32 bits hex string format. EX. “6DA00000” | “00000000” | R/W |
| Valid | String | “Valid” | “0” or “1” | “1” | R/W |

Template Generator

Most attributes in Template Generator JSON content are used to save tool UI settings (except “SelectedFltr”). Those parts have nothing to do with binary creation. For the purpose of binary configuration, those attributes can be kept as it is (default value).

“SelectedFltr” is the only attribute required to set in this section. “SelectedFltr” will define the filters’ type in each template in binary. For the information of filter type, please refer to the “Accepted Value” of “SelectedFltr” in the given table of this section.

Example:

"0": {

"SelectedPie": "PIE Entry",

"InputAIF": "00000000",

"Policer": false,

"IsPolicerEnabled": false,

"InputPolicerVal": "00000",

"SelectedAction": "TOOL_ACL_ACTION_NONE",

"DisplayedAction": "Action",

"SelectedFltr": [

102, 102, 102, 102, 102,

102, 102, 102, 102, 102,

102, 102

],

"DisplayedFltr": [

"--", "--", "--", "--", "--",

"--", "--", "--", "--", "--",

"--", "--"

],

"InputField": [

"0000", "0000", "0000", "0000", "0000",

"0000", "0000", "0000", "0000", "0000",

"0000", "0000"

],

"InputMask": [

"0000", "0000", "0000", "0000", "0000",

"0000", "0000", "0000", "0000", "0000",

"0000", "0000"

],

"InputAIF1": "00000000",

"InputAIF2": "00000000",

"SelectedAction1": "TOOL_ACL_ACTION_NONE",

"SelectedAction2": "TOOL_ACL_ACTION_NONE",

"DisplayedAction1": "Action",

"DisplayedAction2": "Action",

"SelectedDrop": "Bypass",

"SelectedFlow": "Flow 0",

"SelectedMirror": "None",

"SelectedCopy": "None",

"InputCopyText": "0",

},

"1": {

…

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| SelectedFltr | Number | “Filter | 0: "Customized" 1: "DMAC[15:0]" 2: "DMAC[31:16]" 3: "DMAC[47:32] 4: "SMAC[15:0]" 5: "SMAC[31:16]" 6: "SMAC[47:32]" 7: "Ether Type" 8: "OTAG" 9: "ITAG" 12: "SIP[15:0]" 13: "SIP[31:16]" 14: "DIP[15:0]" 15: "DIP[31:16]" 19: "L4 SPORT” 20: "L4 DPORT" 21: "L4 HDR" 100: "SPMASK[12:0]" 102: "--" | 102 | R/W |

GlobalLut

Tool: Switch -> Global Settings -> L2

Example:

"GlobalLut": {

"MulticastHash": true,

"EntryLutList": [

{

"Index": 0,

"MemberPorts": "",

"Type": "Unicast",

"Mac": "10:22:33:44:55:66",

"VId": 2,

"Port": 4,

"ULFId": 1,

"FId": 5,

"IsUnicast": true,

"IsMulticast": false

},

{

"Index": 0,

"MemberPorts": "1,2,3",

"Type": "Multicast",

"Mac": "11:22:33:44:55:66",

"VId": 2,

"Port": 0,

"ULFId": 0,

"FId": 5,

"IsUnicast": false,

"IsMulticast": true

}

]

}

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| MulticastHash | Boolean | “Multicast hash with VID/FID” | true or false | True (Hash with FID) | R/W |
| EntryLutList | Array (JSON Object) | L2 Table | JSON object (Containing Index, MemberPorts, Type, Mac, VId, Port, ULFId, FId, IsUnicast, IsMulticast) | Empty array | R/W |
| Index | Number | “Index” | 0-4095 | 0 | R/W |
| MemberPorts | String | “Member Ports” | Configured member ports in string format (Ex. “1,3,5” or “1-5” or “1,3,5-7”) | empty string | R/W |
| Type | String | “Type” | “Unicast” or “Multicast” | “Unicast” | R/W |
| Mac | String | “MAC Address” | MAC address in string format | “00:00:00:00:00:00” | R/W |
| VId | Number | “VLAN ID” | 1-4095 | 0 | R/W |
| Port | Number | “Port” | 0-11 | 0 | R/W |
| ULFId | Number | “ULFID” | 0 or 1 | 0 | R/W |
| FId | Number | “Filter ID” | 0 - 15 | 0 | R/W |
| IsUnicast | Boolean | Internal use only for debugging |  |  | RO |
| IsMulticast | Boolean | Internal use only for debugging |  |  | RO |

GlobalVlan

Tool: Switch -> Global Settings -> Vlan

Example:

"GlobalVlan": {

"SPID": "88A8",

"TPID": "8100",

"EntryVlanList": [

{

"VId": 2,

"MemberPorts": "1-3",

"UntaggedPorts": "4,5",

"FId": 0,

"SharedVlan": "No",

"VlanBasedFW": "Yes",

"IsRemovable": true,

"IsFIdEnabled": false,

"VlanInterfaceId": 2,

"VlanInterfaceValid": "Valid"

},

{

"VId": 3,

"MemberPorts": "6,7",

"UntaggedPorts": "10",

"FId": 0,

"SharedVlan": "No",

"VlanBasedFW": "Yes",

"IsRemovable": true,

"IsFIdEnabled": false,

"VlanInterfaceId": 3,

"VlanInterfaceValid": "Invalid"

}

]

},

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| SPID | String | "SPID" | 16 bits hex string format | "88A8" | R/W |
| TPID | String | "TPID" | 16 bits hex string format | "8100" | R/W |
| EntryVlanList | Array (JSON Object) | VLAN Table | JSON object (Containing VId, MemberPorts, UntaggedPorts, Fid, SharedVlan, VlanBasedFW, IsRemovable, IsFIdEnabled) | Empty array | R/W |
| VId | Number | “Valid VLAN ID” | 1-4095 | 0 | R/W |
| MemberPorts | String | “Member Ports” | Configured member ports in string format (Ex. “1,3,5” or “1-5” or “1,3,5-7”) | empty string | R/W |
| UntaggedPorts | String | “Untagged Ports” | Configured untagged ports in string format (Ex. “1,3,5” or “1-5” or “1,3,5-7”) | empty string | R/W |
| FId | Number | “FID” | 0-15 | 0 | R/W |
| SharedVlan | String | “Shared Vlan” | “Yes” or “No” | “No” | R/W |
| VlanBasedFW | String | “Vlan Based FW” | “Yes” or “No” | “No” | R/W |
| IsRemovable | Boolean | Internal use only for debugging |  |  | RO |
| IsFIdEnabled | Boolean | Internal use only for debugging |  |  | RO |
| VlanInterfaceId | Number | "Interface ID" | 0-63 | 0 | R/W |
| VlanInterfaceValid | String | "Interface Valid" | "Invalid" or "Valid" | "Invalid" | R/W |

GlobalQos

Tool: Switch -> Global Settings -> QoS
Example:

"GlobalQos": {

"PrioritySource": "QOS_DEFAULT_PRIORITY",

"SelectedPrioritySource": "Default",

"SelectedQueueRemappingPort": "0",

"PriDecQueueList": [{…}],

"PriDecInnerInnerList": [{…}],

"PriDecOuterOuterList": [{…}],

"PriDecDscpDscpList": [{…},{…},{…},{…},{…},{…},{…},{…}],

"PriRmkInnerList": [{…}],

"PriRmkOuterList": [{…}],

"PriRmkDscpList": [{…}]

}

Priority Decision

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| PrioritySource | String | Internal use only for debugging |  |  | RO |
| SelectedPriority- Source | String | “Priority Source” | “Port-based”,"InnerTag-based", "OuterTag-based","Hybrid-based" "DSCP-based","Default" | “Default” | R/W |
| SelectedQueue- RemappingPort | String | “Port” | “0”,”1” ,”2” ,”3” ,”4” ,”5” , ”6” ,”7” ,”8” ,”9” ,”10” ,”11” | “0” | R/W |
| PriDecQueueList | Array (JSON Object) | “Priority Decision” -> “Internal Priority to Queue ID” | JSON objects (Containing Item, Column0,Column1,Column2,Column3, Column4,Column5,Column6,Column7) | JSON Object with default attribute values | R/W |
| PriDecInnerInner- List | Array (JSON Object) | “Priority Decision” (“Inner Tagged Based”) -> “Source Priority to Internal Priority” | JSON objects (Containing Item, Column0,Column1,Column2,Column3, Column4,Column5,Column6,Column7) | JSON Object with default attribute values | R/W |
| PriDecOuterOuter- List | Array (JSON Object) | “Priority Decision” (“Outer Tagged Based”) -> “Source Priority to Internal Priority” | JSON objects (Containing Item, Column0,Column1,Column2,Column3, Column4,Column5,Column6,Column7) | JSON Object with default attribute values | R/W |
| PriDecDscpDscpList | Array (JSON Object) | “Priority Decision” (“DSCP-Based”) -> “Source Priority to Internal Priority” | JSON objects (Containing Item, Column0,Column1,Column2,Column3, Column4,Column5,Column6,Column7) | JSON Object with default attribute values | R/W |

Priority Remarking

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| PriRmkInnerList | Array (JSON Object) | “Priority Remarking” -> “Egress Inner Tag Priority” | JSON objects (Containing Item, Column0,Column1,Column2,Column3, Column4,Column5,Column6,Column7) | JSON Object with default attribute values | R/W |
| PriRmkOuterList | Array (JSON Object) | “Priority Remarking” -> “Egress Outer Tag Priority” | JSON objects (Containing Item, Column0,Column1,Column2,Column3, Column4,Column5,Column6,Column7) | JSON Object with default attribute values | R/W |
| PriRmkDscpList | Array (JSON Object) | “Priority Remarking” -> “Egress Inner DSCP Priority” | JSON objects (Containing Item, Column0,Column1,Column2,Column3, Column4,Column5,Column6,Column7) | JSON Object with default attribute values | R/W |

Priority Decision/Remarking List JSON Object

The example/attribute-table listed below are applied to the JSON objects of:

PriDecQueueList / PriDecInnerInnerList / PriDecOuterOuterList / PriDecDscpDscpList

PriRmkInnerList / PriRmkOuterList / PriRmkDscpList

Example:

"PriDecQueueList": [

{

"Item": "Queue ID (Traffic Class)",

"Column0": 0,

"Column1": 1,

"Column2": 2,

"Column3": 3,

"Column4": 4,

"Column5": 5,

"Column6": 6,

"Column7": 7

}

]

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| Item | String | Priority Decision: “Internal Tag Priority” “Outer Tag Priority” “DSCP Index” Internal Priority To Queue ID Remapping: “Internal Priority” Priority Remarking: “Internal Priority” | Priority Decision: “Internal Priority”, “Internal Priority of DSCP 0-7”, “Internal Priority of DSCP 8-15”, “Internal Priority of DSCP 16-23”, “Internal Priority of DSCP 24-31”, “Internal Priority of DSCP 32-39”, “Internal Priority of DSCP 40-47”, “Internal Priority of DSCP 48-55”, “Internal Priority of DSCP 56-63”, Internal Priority To Queue ID Remapping: “Queue ID (Traffic Class)” Priority Remarking: “Egress Inner Tag Priority” “Egress Outer Tag Priority” “Egress DSCP Priority” | Please refer to default DefaultConfig906x.json file | R/W |
| Column0 | Number | “0” or “+0”(Priority Decision DSCP-based) | 0-7 or 0-63 (Egress DSCP Priority) | Please refer to default DefaultConfig906x.json file | R/W |
| Column1 | Number | “1” or “+1”(Priority Decision DSCP-based) | 0-7 or 0-63 (Egress DSCP Priority) | Please refer to default DefaultConfig906x.json file | R/W |
| Column2 | Number | “2” or “+2”(Priority Decision DSCP-based) | 0-7 or 0-63 (Egress DSCP Priority) | Please refer to default DefaultConfig906x.json file | R/W |
| Column3 | Number | “3” or “+3”(Priority Decision DSCP-based) | 0-7 or 0-63 (Egress DSCP Priority) | Please refer to default DefaultConfig906x.json file | R/W |
| Column4 | Number | “3” or “+3”(Priority Decision DSCP-based) | 0-7 or 0-63 (Egress DSCP Priority) | Please refer to default DefaultConfig906x.json file | R/W |
| Column5 | Number | “4” or “+4”(Priority Decision DSCP-based) | 0-7 or 0-63 (Egress DSCP Priority) | Please refer to default DefaultConfig906x.json file | R/W |
| Column6 | Number | “5” or “+5”(Priority Decision DSCP-based) | 0-7 or 0-63 (Egress DSCP Priority) | Please refer to default DefaultConfig906x.json file | R/W |
| Column7 | Number | “6” or “+6”(Priority Decision DSCP-based) | 0-7 or 0-63 (Egress DSCP Priority) | Please refer to default DefaultConfig906x.json file | R/W |

GlobalRouting

Tool: Switch -> Global Settings -> Routing (Binary Mode Only)

Example:

"GlobalRouting": {

"RoutingInterfaceList": [

{

"Index": 0,

"Ip": "0.0.0.0",

"IpType": "IPv4",

"Mask": "32",

"Port": 0,

"VId": 0,

"Mac": "11:22:00:00:00:00"

}

],

"RoutingHostList": [

{

"Type": "Interface",

"Source": "0.0.0.0",

"Destination": "0.0.0.0",

"IpType": "IPv4",

"SPort": "0",

"DPort": "0",

"InterfaceIdx": "0",

"NextHop": "",

"TTL": "False",

"L2Index": "0",

"Firewall": "False"

}

],

"RoutingStaticList": [

{

"Index": 0,

"Ip": "0.0.0.0",

"IpType": "IPv4",

"Mask": "32",

"NextHopIp": "0.0.0.0"

}

],

"RoutingMultiList": [

{

"Index": 0,

"Sip": "0.0.0.0",

"Dip": "0.0.0.0",

"IpType": "IPv4",

"SPort": "0",

"DPort": "0",

"InterfaceList": "00:00:00:00:00:00:00:07",

"Firewall": "True",

"TTL": "True"

}

],

"RoutingEnable": "True",

"IsVlanSource": "True",

"SelectedACLExtension": "Off"

"TTL": false,

"RoutingFirewall": false,

"DontCare": false,

"downgrade_L2": false,

"RoutingUniList": [

{

"Index": 0,

"Sip": "0.0.0.0",

"Dip": "0.0.0.0",

"IpType": "IPv4",

"SPort": "0",

"DPort": "0",

"L2Index": "0",

"Firewall": "True",

"EgressIdx": "0",

"TTL": "False"

}

],

"RoutingNetworkList": [

{

"Sip": "0.0.0.0",

"Dip": "0.0.0.0",

"IpType": "IPv4",

"SPort": "0",

"DPort": "0",

"L2Index": "0",

"SPortCare": "False",

"DPortCare": "False",

"EgressIdx": "0",

}

],

"RoutingMultiList_image": [],

"RoutingUniList_image": [],

"EgrsMirror_Enbl": false,

"EgrsMirror_Enbl_P8": false,

"EgrsMirror_Enbl_P9": false,

"EgrsMirror_Enbl_txt": "Off",

"SelectedMirrorPort": "1",

"SelectedSparePort": "0"

}

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| RoutingInterfaceList | Array (JSON Object) | “InterfaceList” | JSON object (Containing Index, Ip, IpType, Mask, Port, Vid, Mac) | Empty array | R/W |
| RoutingHostList | Array (JSON Object) | “HostList” | JSON object (Containing Type, Destination, IpType, InterfaceIdx, NextHop) | Empty array | R/W |
| RoutingStaticList | Array (JSON Object) | “StaticList” | JSON object (Containing Index, Ip, IpType, Mask, NextHopIp) | Empty array | R/W |
| RoutingMultiList | Array (JSON Object) | “MultiList” | JSON object (Containing Index, Sip, Dip, IpType, Sport, Dport, InterfaceList, Firewall, TTL) | Empty array | R/W |
| RoutingUniList | Array (JSON Object) | “Static Host List” | JSON object (Containing Index, Sip, Dip, IpType, Sport, Dport, L2Index, Firewall, EgressIdx, TTL) | Empty array | R/W |
| RoutingEnable | Boolean | “Routing Enable” | True or false | False | R/W |
| IsVlanSource | Boolean | “IsVlanSource” | True or false | False | R/W |
| SelectedACLExtension | String | "Selected ACL Extension" | "Off" or "On" | "Off" | R/W |
| TTL | Boolean | “TTL” | True or false | false | R/W |
| RoutingFirewall | Boolean | “Firewall” | True or false | False | R/W |
| DontCare | Boolean | Internal use only for debugging |  |  | RO |
| downgrade_L2 | Boolean | "L2 Downgrade" | True or false | false | R/W |
| RoutingNetworkList | Array (JSON Object) | “Routing Network List” | JSON object (Containing Sip, Dip, IpType, SPort, DPort, L2Index, SPortCare, DPortCare, EgressIdx) | Empty array | R/W |
| RoutingMultiList_image | Array (JSON Object) | Internal use only for debugging |  |  | RO |
| RoutingUniList_image | Array (JSON Object) | Internal use only for debugging |  |  | RO |

RoutingInterfaceList (Interface Config)

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| Index | Number | “Index” | 0-63 | 0 | R/W |
| Ip | String | “IP” | IP address in String format | “0.0.0.0” | R/W |
| IpType | string | “IP Type” | “IPv4”, “IPv6” | “IPv4” | R/W |
| Mask | String | “Mask” | Mask(32 bits) in String format | “32” | R/W |
| Port | Number | “Port” | 0-11 | 0 | R/W |
| Vid | Number | “VID” | 0-4095 | 0 | R/W |
| Mac | String | “MAC” | MAC address in String format | “00:00:00:00:00:00” | R/W |

RoutingStaticList (Static Route)

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| Index | Number | Internal use only for debugging |  |  | RO |
| Ip | String | “IP” | IP address in String format | “0.0.0.0” | R/W |
| IpType | string | “IP Type” | “IPv4”, “IPv6” | “IPv4” | R/W |
| Mask | String | “Mask” | Mask(32 bits) in String format | “32” | R/W |
| NextHopIp | String | “Next Hop” | IP address in String format | “0.0.0.0” | R/W |

RoutingMultiList (Multicast)

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| Index | Number | Internal use only for debugging |  |  | RO |
| Sip | String | “SIP” | IP address in String format | “0.0.0.0” | R/W |
| Dip | string | “DIP” | IP address in String format | “0.0.0.0” | R/W |
| IpType | String | “IP Type” | “IPv4”, “IPv6” | “IPv4” | R/W |
| Sport | Number | “Sport” | 0-65535 | 0 | R/W |
| Dport | Number | “Dport” | 0-65535 | 0 | R/W |
| InterfaceList | String | “Interface List” | Mask in String format EX. “01:01:01:01:01:01:01:01” | No Default Value | R/W |
| Firewall | Boolean | “Firewall” | True or false | false | R/W |
| TTL | Boolean | “Interface List” | True or false | false | R/W |

RoutingUniList (Global Routing Table)

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| Index | Number | Internal use only for debugging |  |  | RO |
| Sip | String | “SIP” | IP address in String format | “0.0.0.0” | R/W |
| Dip | string | “DIP” | IP address in String format | “0.0.0.0” | R/W |
| IpType | String | “IP Type” | “IPv4”, “IPv6” | “IPv4” | R/W |
| Sport | Number | “Sport” | 0-65535 | 0 | R/W |
| Dport | Number | “Dport” | 0-65535 | 0 | R/W |
| InterfaceList | String | “Interface List” | Mask in String format EX. “01:01:01:01:01:01:01:01” | No Default Value | R/W |
| Firewall | Boolean | “Firewall” | True or false | false | R/W |
| EgressIdx | Number | “EgressIdx” | 0-63 | 0 | R/W |
| TTL | Boolean | “TTL” | True or false | false | R/W |

RoutingHostList (Static Host)

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| Type | String | “Type” | “Interface”, “Static”, “Multi” | No Default Value | R/W |
| Source | String | “Source” | “IPv4”, “IPv6” | No Default Value | R/W |
| Destination | String | “Destination” | Combination of IP and subnet mask in String format EX. “192.168.1.1/32” | No Default Value | R/W |
| InterfaceIdx | String | “Index” | If “Type” is “Interface”, this value must be set to certain “index” of RoutingInterfaceList entry. If “Type” is “Multi”, this value must be set to certain “InterfaceList” of RoutingMultiList entry. If “Type” is “Static” or “Static Host”, this value must be set to empty String “”. | No Default Value | R/W |
| NextHop | String | “Next Hop” | IP address in String format | No Default Value | R/W |
| Sport | Number | “Sport” | 0-65535 | No Default Value | R/W |
| Dport | Number | “Dport” | 0-65535 | No Default Value | R/W |
| L2Index | Number | “L2Index” | 0-65535 | No Default Value | R/W |
| ConfigInfo | Number | “ConfigInfo” | 0-65535 | No Default Value | R/W |

RoutingNetworkList (Network List)

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| Sip | String | “SIP” | IP address in String format | “0.0.0.0” | R/W |
| Dip | string | “DIP” | IP address in String format | “0.0.0.0” | R/W |
| IpType | String | “IP Type” | “IPv4”, “IPv6” | “IPv4” | R/W |
| SPort | Number | “Sport” | 0-65535 | No Default Value | R/W |
| DPort | Number | “Dport” | 0-65535 | No Default Value | R/W |
| L2Index | Number | “L2Index” | 0-65535 | No Default Value | R/W |
| SPortCare | Number | “Dport” | 0-65535 | No Default Value | R/W |
| DPortCare | Number | “Dport” | 0-65535 | No Default Value | R/W |
| EgressIdx | Number | “EgressIdx” | 0-63 | 0 | R/W |

GlobalFlowControl

Tool: Switch -> Global Settings -> Flow Ctrl

Example:

"GlobalFlowControl": {

"FlowEnable": false,

"PortStatus": [

false, false, false, false, false,

false, false, false, false, false,

false, false, false, false, false

]

}

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| FlowEnable | Boolean | "Flow Control Configuration Setting" | true or false | false | R/W |
| PortStatus | Array (Boolean) | "Port 0" - "Port 14" | Array of 15 Booleans | Array of 15 false | R/W |

GlobalFrer

Tool: Switch -> Global Settings -> FRER

Example:

"GlobalFrer": {

"StreamStartIndex": 1,

"StreamEndIndex": 15,

"EntryStreamList": [

{

"EntryIndex": 1,

"Valid": false,

"DestAddr": "0x0",

"SrcAddr": "0x0",

"Ethertype": "0x0",

"VlanID": "0",

"SrcIP": "0x0",

"DestIP": "0x0",

"DSCP": "0x0",

"NextProtocol": "0x0",

"SrcPort": "0",

"DestPort": "0",

"PortMask": "0x0",

"StreamHandle": "0x0"

}

]

"CountersStartIndex": 1,

"CountersEndIndex": 15

}

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| StreamStartIndex | Number | "Stream Identification" -> "Start" | 1-96 | 1 | R/W |
| CountersEndIndex | Number | "Stream Identification" -> "End" | 1-96 | 15 | R/W |
| EntryStreamList | Array (JSON Object) | "EntryStreamList" | JSON object (Containing "EntryIndex", "Valid", "DestAddr", "SrcAddr", "Ethertype", "VlanID", "SrcIP", "DestIP", "DSCP", "NextProtocol", "SrcPort", "DestPort", "PortMask" and "StreamHandle") | Empty array | R/W |
| EntryIndex | Number | "Entry" | 1-96 | No default value | R/W |
| Valid | Boolean | "Valid" | true or false | false | R/W |
| DestAddr | String | "Destination Address" | Hex integer string | No default value | R/W |
| SrcAddr | String | "Source Address" | Hex integer string | No default value | R/W |
| Ethertype | String | "Ethertype" | Hex integer string | No default value | R/W |
| VlanID | String | "VLAN ID" | Integer string | No default value | R/W |
| SrcIP | String | "Source IP" | Hex integer string | No default value | R/W |
| DestIP | String | "Destination IP" | Hex integer string | No default value | R/W |
| DSCP | String | "DSCP" | Hex integer string | No default value | R/W |
| NextProtocol | String | " Next Protocol" | Hex integer string | No default value | R/W |
| SrcPort | String | "Source Port" | Integer string | No default value | R/W |
| DestPort | String | "Destination Port" | Integer string | No default value | R/W |
| PortMask | String | "Port Mask" | Hex integer string | No default value | R/W |
| StreamHandle | String | " Stream Handle" | Hex integer string | No default value | R/W |
| CountersStartIndex | Number | "Counters" -> "Start" | 1-96 | 1 | R/W |
| CountersStartIndex | Number | "Counters" -> "End" | 1-96 | 15 | R/W |

GlobalPsfp

Tool: Switch -> Global Settings -> PSFP

Example:

"GlobalFrer": {

"StreamFilterStartIndex": 0,

"StreamFilterEndIndex": 15,

"EntryStreamFilterList": [

{

"EntryIndex": 0,

"Valid": false,

"MatchingStreamHandle": "0x0",

"MatchingPriority": "0",

"StreamGateID": "0",

"FlowMeterID": "0",

"MaxSDUsize": "0",

"BlockStreamEnable": "Disabled"

}

],

"StreamGateStartIndex": 0,

"StreamGateEndIndex": 15,

"EntryStreamGateList": [

{

"EntryIndex": 0,

"Valid": false,

"AdminGateState": "Closed",

"AdminIPV": "Bypass",

"GateControlListEnable": "Disabled",

"GateClose_RX": "Disabled"

}

],

"EntryGclWindow": {

"0"-"95": {

"BaseTime_MsbSec": "0x0",

"BaseTime_LsbSec": "0x0",

"BaseTime_ns": "0x0",

"CycleTimeEtn": "0x0",

"SelectedGclLength": 2,

"GateClose_Octet": "Disabled",

"EntryStreamGateList": [

{

"StreamGateIndex": 0,

"ListIndex": 0,

"GateState": "Open",

"IPV": "Bypass",

"Duration": "0",

"IntervalOctets": "0"

}

]

}

},

"FlowMeterStartIndex": 0,

"FlowMeterEndIndex": 15,

"EntryFlowMeterList": [

{

"EntryIndex": 0,

"Valid": false,

"CommitInfoRate": "",

"CommitBurstSize": "",

"ExcessInfoRate": "",

"ExcessBurstSize": "",

"CouplingFlag": "False",

"ColorMode": "ColorBlind",

"DeiSelection": "InnerTag",

"DropOnYellowFrame": "False",

"IPVforYellowFrame": "Disabled",

"MarkAllFramesRed": "Disabled"

}

],

"CountersStartIndex": 0,

"CountersEndIndex": 15,

"ErrorStartIndex": 0,

"ErrorEndIndex": 15

}

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| StreamFilterStartIndex | Number | "Stream Filter" -> "Start" | 0-95 | 0 | R/W |
| StreamFilterEndIndex | Number | "Stream Filter" -> "End" | 0-95 | 15 | R/W |
| EntryStreamFilterList | Array (JSON Object) | "Stream Filter" | JSON object (Containing "EntryIndex", "Valid", "MatchingStreamHandle", "MatchingPriority", "StreamGateID", "FlowMeterID", "MaxSDUsize" and "BlockStreamEnable") | Empty array | R/W |
| EntryIndex | Number | "Entry" | 0-95 | No default value | R/W |
| Valid | Boolean | "Valid" | true or false | false | R/W |
| MatchingStreamHandle | String | "Matching Stream Handle" | Hex integer string | No default value | R/W |
| MatchingPriority | String | "Matching Priority" | Integer string | No default value | R/W |
| StreamGateID | String | "Stream Gate ID" | Integer string | No default value | R/W |
| FlowMeterID | String | "Flow Meter ID" | Integer string | No default value | R/W |
| MaxSDUsize | String | "Max SDU size in Byte" | Integer string | No default value | R/W |
| BlockStreamEnable | String | "Block Stream due to Oversize" | "Disabled" or "Enabled" | "Disabled" | R/W |
| StreamGateStartIndex | Number | "Stream Gate" -> "Start" | 0-95 | 0 | R/W |
| StreamGateEndIndex | Number | "Stream Gate" -> "End" | 0-95 | 15 | R/W |
| EntryStreamGateList | Array (JSON Object) | "Stream Gate" | JSON object (Containing "EntryIndex", "Valid", "AdminGateState", "AdminIPV", "GateControlListEnable" and "GateClose_RX") | Empty array | R/W |
| EntryIndex | Number | "Entry" | 0-95 | 0 | R/W |
| Valid | Boolean | "Valid" | true or false | false | R/W |
| AdminGateState | String | "Admin Gate State" | "Closed" or "Open" | "Closed" | R/W |
| AdminIPV | String | "Admin IPV" | "1"-"7" and "Bypass" | "Bypass" | R/W |
| GateControlListEnable | String | "Gate Control List" | "Disabled" or "Enabled" | "Disabled" | R/W |
| GateClose_RX | String | "Gate Close due to Invalid RX" | "Disabled" or "Enabled" | "Disabled" | R/W |
| EntryGclWindow | Array (JSON Object) | "0"-"95" "Stream Gate" -> "Gate Control List Set" | JSON object (Containing "BaseTime_MsbSec", "BaseTime_LsbSec", "BaseTime_ns", "CycleTimeEtn", "SelectedGclLength", "GateClose_Octet" and "EntryGclList") | Empty array | R/W |
| BaseTime_MsbSec | String | "Stream Gate" -> "Gate Control List Set" -> "Base Time" | Hex integer string | No default value | R/W |
| BaseTime_LsbSec | String | "Stream Gate" -> "Gate Control List Set" -> "Base Time" | Hex integer string | No default value | R/W |
| BaseTime_ns | String | "Stream Gate" -> "Gate Control List Set" -> "Base Time" | Hex integer string | No default value | R/W |
| CycleTimeEtn | String | "Stream Gate" -> "Gate Control List Set" -> "Cycle Time Extension" | Hex integer string | No default value | R/W |
| SelectedGclLength | Number | "Stream Gate" -> "Gate Control List Set" -> "Control List Length" | "2"-"8" | No default value | R/W |
| GateClose_Octet | String | "Stream Gate" -> "Gate Control List Set" -> "Gate Close due to Exceed Octet" | "Disabled" or "Enabled" | "Disabled" | R/W |
| EntryGclList | Array (JSON Object) | "Stream Gate" -> "Gate Control List Set" -> "Gate Control List" | JSON object (Containing "StreamGateIndex", "ListIndex", "GateState", "IPV", "Duration" and "IntervalOctets") | Empty array | R/W |
| StreamGateIndex | Number | Internal use only for debugging |  |  | RO |
| ListIndex | Number | "Index" | 0-1 | 0 | R/W |
| GateState | String | "Gate State" | "Closed" or "Open" | "Open" | R/W |
| IPV | String | "IPV" | "1"-"7" and "Bypass" | "Bypass" | R/W |
| Duration | String | "Duration" | Integer string | "0" | R/W |
| IntervalOctets | String | "Interval Octets" | Integer string | "0" | R/W |
| FlowMeterStartIndex | Number | "Flow Meter" -> "Start" | 0-95 | 0 | R/W |
| FlowMeterEndIndex | Number | "Flow Meter" -> "End" | 0-95 | 15 | R/W |
| EntryFlowMeterList | Array (JSON Object) | "Flow Meter" | JSON object (Containing "EntryIndex", "Valid", "CommitInfoRate", "CommitBurstSize", "ExcessInfoRate", "ExcessBurstSize", "CouplingFlag", "ColorMode", "DeiSelection", "DropOnYellowFrame", "IPVforYellowFrame" and"MarkAllFramesRed") | Empty array | R/W |
| "EntryIndex" | Number | "EntryIndex" | 0-95 | 0 | R/W |
| "Valid" | Boolean | "Valid" | true or false | false | R/W |
| "CommitInfoRate" | String | "CommitInfoRate" | Integer string | "0" | R/W |
| "CommitBurstSize" | String | "CommitBurstSize" | Integer string | "0" | R/W |
| "ExcessInfoRate" | String | "ExcessInfoRate" | Integer string | "0" | R/W |
| "ExcessBurstSize" | String | "ExcessBurstSize" | Integer string | "0" | R/W |
| "CouplingFlag" | String | "CouplingFlag" | "True" or "False" | "False" | R/W |
| "ColorMode" | String | "ColorMode" | "ColorBlind" or "ColorAware" | "ColorBlind" | R/W |
| "DeiSelection" | String | "DeiSelection" | "InnerTag" or "OuterTag" | "InnerTag" | R/W |
| "DropOnYellowFrame" | String | "DropOnYellowFrame" | "True" or "False" | "False" | R/W |
| "IPVforYellowFrame" | String | "IPVforYellowFrame" | "Disabled" or "Enabled" | "Disabled" | R/W |
| "MarkAllFramesRed" | String | "MarkAllFramesRed" | "Disabled" or "Enabled" | "Disabled" | R/W |
| CountersStartIndex | Number | "Counters" -> "Start" | 0-95 | 0 | R/W |
| CountersEndIndex | Number | "Counters" -> "End" | 0-95 | 15 | R/W |
| ErrorStartIndex | Number | "Counters" -> "Start" | 0-95 | 0 | R/W |
| ErrorEndIndex | Number | "Counters" -> "End" | 0-95 | 15 | R/W |

GlobalPhyMaster

Tool: Switch -> Global Settings -> Phy Master

Example:

"GlobalFrer": {

"InputPhyID": [

1, 2, 3, 4

]

"SelectPower_Rate": 5,

"SelectedPhyIdMode": "PHY 0",

"PhyAddr": "",

"PhyVal": "",

"SelectedMappingMode": "PHY 0",

"Mapping_Page": "",

"Mapping_Reg": "",

"Mapping_Ext": ""

}

- 2.17 GlobalIgmp

"GlobalIgmp": {

"IGMP": false,

"MLD": false,

"FastLeave": true,

"VLAN": true,

"LastMem": "2",

"GroupMem": "100",

"L2Drop": false,

"L2Customized": false

}

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| InputPhyID | Array (Number) | "Phy 0 ID", "Phy 1 ID", "Phy 2 ID" and "Phy 3 ID" | Integer number | 1, 2, 3, 4 | R/W |
| SelectPower_Rate | Number | "Frequency" | Integer number | 5 | R/W |
| SelectedPhyIdMode | String | "Which Phy to initailize" | "PHY 0", " PHY 1", "PHY 2" and " PHY 3" | "PHY 0" | R/W |
| PhyAddr | Number | "Register Address" | Hex integer number | No default value | R/W |
| PhyVal | Number | "Register Value" | Hex integer number | No default value | R/W |
| SelectedMappingMode | String | "Which Phy" | "PHY 0", " PHY 1", "PHY 2" and " PHY 3 | "PHY 0" | R/W |
| Mapping_Page | Number | "Page" | Hex integer number | No default value | R/W |
| Mapping_Reg | Number | "Reg" | Integer number | No default value | R/W |
| Mapping_Ext | Number | "External PHY Reg" | Integer number | No default value | R/W |

Port

PortGeneral

Tool: Switch -> P0-P14 -> Properties

Example:

"PortGeneral": {

"Role": true,

"SelectedPortIf": "RGMII",

"PhyStatus": true,

"Half_100m": false,

"Half_10m": false,

"Full_100m": false,

"Full_10m": false,

"isAutoNego": false,

"isForce": false,

"isSerdes": false,

"SelectedLinkSpeed": "1000M",

"RxDelay": 0,

"TxDelay": 0,

"SelectedLdoVoltage": "1.8v",

"SelectedRGMIIVoltage": "3.3v",

"SelectedSpeed": "10mbps",

"RGMIIDriving": "0",

"RGMIIDrivingLow": "0",

"DrivingEnable": "True"

"SelectedSgmiiExternalPortCfg": "CML/N-Way enable PHY side, speed 1000Mbps",

"Rolelabel": "Master",

"DrivingEnablelabel": "Auto"

"isSerdeslabel": "MAC",

"isForcelabel": "OFF"

}

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| SelectedPortIf | String | “Interface” | “100BASE T1 PHY”, “1000BASE T1 PHY”, ”RGMII”, ”SGMII”, ”RGMII PHY”, ”RGMII MAC”, ”MII PHY” , ”MII MAC” , ”RGMII 100M”, ”HISGMII”, ”USXGMII”, ”FAST ETHERNET”, ”PCIE1G” ”PCIE5G” * Supported interfaces are varied from port to port, please check RTL907X_6XA-V1 Series Datasheet | Please refer to default DefaultConfig906x.json file | R/W |
| Role | Boolean | “Role” | True(Master) or false(Slave) *Only affects when interface is “100BASE T1 PHY” or “1000BASE T1 PHY” | true | R/W |
| PhyStatus | Boolean | Internal use only for debugging |  |  | RO |
| isForce | Boolean | "Force Mode" | true or false | false | R/W |
| isSerdes | Boolean | "Serdes Mode" | true or false | false | R/W |
| SelectedLinkSpeed | string | "Link Speed" | RGMII: 1000M/100M/10M RMII/MII PHY/MAC: 100M/10M | RGMII: 1000M RMII/MII PHY/MAC: 100M | R/W |
| RxDelay | Number | “RGMII RX Delay” | 0-7 *Only affects when interface is “RGMII” | 0 | R/W |
| TxDelay | Number | “RGMII TX Delay” | 0-1 *Only affects when interface is “RGMII” | 0 | R/W |
| SelectedLdoVoltage | string | “xMII LDO” | “1.25v”,”1.8v”,”Disable” * Only P4,P5,P9(9072A) or P0,P1,P5(9068A) have this attribute | “1.8v” | R/W |
| SelectedRGMIIVoltage | string | “xMII Voltage” | “3.3v”,“2.5v”,”1.8v” * Only P4,P5,P8,P9,P11(9072A/9075A) or P0,P1,P5,P6,P8(9068A) have this attribute | “1.8v” | R/W |
| SelectedSpeed | string | "Speed" | SGMII: "10mbps", "100mbps" and "1000mbps" | "10mbps" | R/W |
| RGMIIDriving | Number | “Rising Strength” | 0-7 | 0 | R/W |
| RGMIIDrivingLow | Number | “Falling Strength” | 0-7 | 0 | R/W |
| DrivingEnable | Boolean | “xMII Driving” | True or false | false | R/W |
| SelectedSgmiiExternalPortCfg | string | Internal use only for debugging |  |  | RO |
| Rolelabel | string | Internal use only for debugging |  |  | RO |
| DrivingEnablelabel | string | Internal use only for debugging |  |  | RO |
| isSerdeslabel | string | Internal use only for debugging |  |  | RO |
| isForcelabel | string | Internal use only for debugging |  |  | RO |
| Half_100m | Boolean | "100M Half" | True or false | false | R/W |
| Half_10m | Boolean | "100M Full" | True or false | false | R/W |
| Full_100m | Boolean | "10M Half" | True or false | false | R/W |
| Full_10m | Boolean | "10M Full" | True or false | false | R/W |
| isAutoNego | Boolean | "Auto Negotiation" | True or false | false | R/W |

PortVlan

Tool: Switch -> P0-P14 -> Vlan

Example:

"PortVlan": {

"PortNum": 2,

"InputVId": 1,

"InputPriority": 0,

"SelectedLearningBased": "Inner Tag",

"SelectedMode": "802.1Q-based",

"IngressFilter": true,

"EgressFilter": true,

"SelectedFrameType": "ACCEPT_FRAME_TYPE_ALL",

"DisplayedFrameType": "Untagged, priority-tagged and tagged"

}

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| PortNum | Number | Internal use only for debugging |  |  | RO |
| InputVId | Number | “VID” | 0-4095 | 1 | R/W |
| InputPriority | Number | “Priority” | 0-7 | 0 | R/W |
| SelectedLearningBased | String | “VLAN learning based on” | “Inner Tag”, ”Outer Tag” | Inner Tag | R/W |
| SelectedMode | String | “VLAN Mode” | “802.1Q-Based”,”Port-Based” | 802.1Q-Based | R/W |
| IngressFilter | Boolean | “Ingress Filter” | True(Enabled) or false(Disabled) | True | R/W |
| EgressFilter | Boolean | “Egress Filter” | True(Enabled) or false(Disabled) | True | R/W |
| IngressFilterLabel | String | Internal use only for debugging |  |  | RO |
| EgressFilterLabel | String | Internal use only for debugging |  |  | RO |
| DisplayedFrameType | String | Internal use only for debugging |  |  | RO |
| SelectedFrameType | String | “Accepted Frame Type” | “Untagged,priority-tagged and tagged”, “Tagged only”, “Untagged and Priority Tagged only”, ”None” | “Untagged,priority-tagged and tagged” | R/W |

PortQos

Tool: Switch -> P0-P14 -> QoS

Example:

"PortQos": {

"PriDecPriority": 2,

"PriRmkInnerTag": "Disabled",

"PriRmkOuterTag": "Disabled",

"PriRmkDscp": "Disabled",

"PriRmkBoundaryPort": "Disabled",

"PriRmkBoundaryPortList": [

{

"Item": "Remark to PCP",

"Column0": 0,

"Column1": 0,

"Column2": 0,

"Column3": 0,

"Column4": 0,

"Column5": 0,

"Column6": 0,

"Column7": 0

}

],

"SelectedQueueSchedComb": "Strict Priority + WRR",

"DisplayedSchedItem0": "Strict Priority",

"DisplayedSchedItem1": "WRR",

"QueueSchedItem0": [

"SP_0", "SP_1", "SP_2", "SP_3", "SP_4",

"SP_5", "SP_6", "SP_7"

],

"QueueSchedItem1": [

"WRR_0", "WRR_1", "WRR_2", "WRR_3", "WRR_4",

"WRR_5", "WRR_6", "WRR_7" ],

"SelectedQueueSched": [

"Strict Priority", "Strict Priority", "Strict Priority", "Strict Priority", "Strict Priority",

"Strict Priority", "Strict Priority", "Strict Priority" ],

"IsQueueWeightEnabled": [

false, true, true, true, true,

true, true, true

],

"IsRateLimitingEnabled": [

true, true, true, true, true,

true, true, true

],

"IsRateEnableOn": [

false, false, false, false, false,

false, false, false

],

"InputQueueWeight": [

0, 2, 4, 8, 1,

2, 4, 8

],

"InputRateLimiting": [

1048568, 1048568, 1048568, 1048568, 1048568,

1048568, 1048568, 1048568

],

"IsCbsBwEnabled": [

false, false, false, false, false,

false, false, false

],

"InputCbsBw": [

0, 0, 0, 0, 0,

0, 0, 0

]

}

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| PriDecPriority | Number | “Priority Decision” -> “Port-Based Priority” | 0-7 | 2 | R/W |
| PriRmkInnerTag | String | “Priority Remarking” -> “Inner Tag Priority Remarking” | “Disable”, ”Enable” | Disable | R/W |
| PriRmkOuterTag | String | “Priority Remarking” -> “Outer Tag Priority Remarking” | “Disable”, ”Enable” | Disable | R/W |
| PriRmkDscp | String | “Priority Remarking” -> “DSCP Remarking” | “Disable”, ”Enable” | Disable | R/W |
| SelectedQueueSchedComb | String | “Queue Settings” -> “Queue Schedulers” | “Strict Priority + WRR”, “Strict Priority + WFQ” | “Strict Priority + WRR” | R/W |
| DisplayedSchedItem0 | String | Internal use only for debugging |  |  | RO |
| DisplayedSchedItem1 | String | Internal use only for debugging |  |  | RO |
| QueueSchedItem0 | String | Internal use only for debugging |  |  | RO |
| QueueSchedItem1 | String | Internal use only for debugging |  |  | RO |
| SelectedQueueSched | Array (String) | “Queue Settings” -> “Scheduler” | “Strict Priority”, “WRR”, ”WFQ” | “Strict Priority” | R/W |
| IsQueueWeightEnabled | Array (Boolean) | Internal use only for debugging |  |  | RO |
| IsRateLimitingEnabled | Array (Boolean) | Internal use only for debugging |  |  | RO |
| IsRateEnableOn | Array (Boolean) | Internal use only for debugging |  |  | RO |
| InputQueueWeight | Array (Number) | “Weight” | 0-63 | Please refer to default DefaultConfig906x.json file | R/W |
| InputRateLimiting | Array (Number) | “Rate” | 0-1048568 | 1048568 | R/W |
| IsCbsBwEnabled | Array (Boolean) | "Queue Settings" -> "Queue Schedulers" -> "Strict Priority + CBS" | true or false | True | R/W |
| InputCbsBw | Array (Number) | "CBS Bandwidth" | 0-5000000000 | 0 | R/W |

PortTraffic

Tool: Switch -> P0-P14 -> Traffic Scheduler

Example:

"PortTraffic": {

"AdminGateState": "0xFF",

"OperGateState": "",

"GateEnableState": "Disabled",

"QueueMaxSDU": [

"0x3FFF", "0x3FFF", "0x3FFF", "0x3FFF", "0x3FFF",

"0x3FFF", "0x3FFF", "0x3FFF"

],

"BaseTime_MsbSec": "0x0",

"BaseTime_LsbSec": "0x0",

"BaseTime_ns": "0x0",

"CycleTimeEtn": "0x0",

"ControlListLength": 2,

"EntryTrafficList": [

{

"Index": 0,

"GateState": "",

"Interval": ""

},

{

"Index": 1,

"GateState": "",

"Interval": ""

}

],

"CycleTime": "-",

"SelectedPortType": "100M"

}

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| AdminGateState | String (String) | "Admin Gate State" | hex string format | "0xFF" | R/W |
| OperGateState | String (String) | "Oper Gate State" | "Disable" and "Enable" | "Disable" | R/W |
| GateEnableState | Boolean | "GateEnableState" | "Disable" and "Enable" | "Disable" | R/W |
| QueueMaxSDU | Array (String) | Queue Max SDU | 16 bits hex string format | "0x3FFF" | R/W |
| BaseTime_MsbSec | String (String) | "Base Time" | hex string format | "0x0" | R/W |
| BaseTime_LsbSec | String (String) | "Base Time" | hex string format | "0x0" | R/W |
| BaseTime_ns | String (String) | "Base Time" | hex string format | "0x0" | R/W |
| CycleTimeEtn | String (String) | "Cycle Time Extension" | hex string format | "0x0" | R/W |
| ControlListLength | Number | "Control List Length" | integer | 2 | R/W |
| EntryTrafficList | Array (JSON Object) | “Gate Control List” | JSON object (Containing Index, GateState, Interval) | Empty array | R/W |
| Index | Number | "Gate Control List" -> "index" | integer | 0 | R/W |
| GateState | String (String) | "Gate Control List" -> "gate state" | "Disable" and "Enable" | "Disable" | R/W |
| Interval | String (String) | "Gate Control List" -> "interval" | hex string format | "0" | R/W |
| CycleTime | String (String) | "Cycle Time" | hex string format | "-" | R/W |
| SelectedPortType" | String (String) | "Port Type" | String | "100M" | R/W |

PortStormControl

Tool: Switch -> P0-P14 -> Storm Control

Example:

"PortStormControl": {

"BroadcastStormControl": "Disabled",

"BcDataRateType": "bps",

"BroadcastDataRate": "1",

"MulticastStormControl": "Disabled",

"McDataRateType": "bps",

"MulticastDataRate": "1",

"SelectedMulticastType": "Unknown Multicast Only",

"UnicastStormControl": "Disabled",

"UcDataRateType": "bps",

"UnicastDataRate": "1"

}

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| BroadcastStormControl | String | "Broadcast" -> "StormControl" | "Enabled" or "Disabled" | "Disabled" | R/W |
| BcDataRateType | String | "Broadcast" -> "Data Rate" | "bps" or "pps" | "bps" | R/W |
| BroadcastDataRate | Number | "Broadcast" -> "Data Rate" | Integer | 0 | R/W |
| MulticastStormControl | String | "Multicast" -> "StormControl" | "Enabled" or "Disabled" | "Disabled" | R/W |
| McDataRateType | String | "Multicast" -> "Data Rate" | "bps" or "pps" | "bps" | R/W |
| MulticastDataRate | Number | "Multicast" -> "Data Rate" | Integer | 0 | R/W |
| SelectedMulticastType | String | "Multicast" -> "Multicast Type" | "Unknown Multicast Only" or "All Multicast" | "Unknown Multicast Only" | R/W |
| UnicastStormControl | String | "Unicast" -> "StormControl" | "Enabled" or "Disabled" | "Disabled" | R/W |
| UcDataRateType | String | "Unicast" -> "Data Rate" | "bps" or "pps" | "bps" | R/W |
| UcDataRateType | Number | "Unicast" -> "Data Rate" | Integer | 0 | R/W |

PortIsolation

Tool: Switch -> P0-P14 -> Port Isolation

Example:

"PortIsolation": {

"BlockedFwPortMask": [

false, false, false, false, false,

false, false, false, false, false,

false, false, false, false, false

]

}

| Key | Type | Items On Tool | Accepted Value | Default | R/W |
| --- | --- | --- | --- | --- | --- |
| BlockedFwPortMask | Array (Boolean) | "Blocked Forwarding Ports" | Array of 15 Booleans | Array of 15 false | R/W |